| Hangarvej B 2 DK-4000 Roskilde Danmark CVR:49667728. Ph. Office: +45 46 19 11 00; Fax:+45 46 19 11 15. Flyservice@aircat.dk. | Projection List OY-BUF | Actions. Rep - Replacement. Ins - Inspection. |
| --- | --- | --- |

| Created from the following values |  |  |
| --- | --- | --- |
| Aircraft Time. Date; maj 5,2027. TTAF; 5 373.5. | Custom Cycle. Name; Total. Flight Segment; 0.0. Landings; 6 394.0. Lifts; 0.0. | Meter data. Name; Time; Cycle. |

| Components |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | Components/Action | Serial No. | Limit | Perf. | TTSC | TTSN | Due | Remaining |
| OY-BUF | CO Detector / Rep |  | 12.00M | 4820.30H apr 3, 2025 | 553.20H 25.07M |  | apr 1, 2027 | -1.13M |
| OY-BUF | Induction Air Filter / Rep |  | 200.00H 12.00M | 4869.28H apr 29, 2026 | 504.22H 12.20M |  | 5069.28H apr 29, 2027 | -304.22H -0.20M |
| OY-BUF | Instrument filter / Rep |  | 500.00H | 4579.60H sep 25, 2020 | 793.90H 79.33M |  | 5079.60H | -293.90H |
| OY-BUF | Sparkplugs / Rep |  | 400.00H | 4653.20H mar 22, 2022 | 720.30H 61.43M |  | 5053.20H | -320.30H |
| OY-BUF | Vacuum pump / Tolerance ±10% |  | 72.00M | 4590.90H mar 30, 2021 | 782.60H 73.17M |  | mar 30, 2027 | -1.17M |

| Tasks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | Task # | Task | ATA Code | Action | Limit | Perf. | TTSC | Due | Rem. |
| OY-BUF | Master 11 | Annual inspection of ELT | 2560 | Ins | 12.00M | 4869.28H apr 29, 2026 | 504.22H 12.20M | apr 29, 2027 | -0.20M |
| OY-BUF Tolerance ±0% | Master 10 | Check of emergency equipment FIRST AID KIT, Extinguishers, Life Jacket | 2500 | Ins | 12.00M | 4869.28H apr 29, 2026 | 504.22H 12.20M | apr 29, 2027 | -0.20M |
| OY-BUF | Master 12 | Engine - Valve Inspection - Remove rocker box covers and check for freedom | 7100 | Ins | 400.00H | 4653.20H mar 22, 2022 | 720.30H 61.43M | 5053.20H | -320.30H |
| OY-BUF | Master 13 | Equipment Visual Inspection: A visual inspection of the equipment installed by this STC must be performed. | 2320 | Ins | 12.00M | 4869.28H apr 29, 2026 | 504.22H 12.20M | apr 29, 2027 | -0.20M |
| OY-BUF Tolerance ±10% | Master 01 | LH magneto. 500 hrs insp. on component: Magneto (L/H) Make:SLICK/CHAMPION Model: Part Number:4371 Serial Number:J-08027 Tracking #:19494 | 7400 | Ins | 500.00H | 4653.20H mar 22, 2022 | 720.30H 61.43M | 5153.20H | -220.30H |
| OY-BUF Tolerance ±10% | 5 | Perform vacuum pump vane wear inspection. on component: Vacuum pump Part Number:7.7511B-1 Serial Number: Tracking #:6910 | 3700 | Ins | 100.00H | 4869.28H apr 29, 2026 | 504.22H 12.20M | 4969.28H | -404.22H |
| OY-BUF Tolerance ±10% | Master 03 | RH magneto. 500 hrs insp. on component: Magneto (R/H) Make:Slick/Champion Model: Part Number:4370 Serial Number:J-07470 Tracking #:19495 | 7400 | Ins | 500.00H | 4653.20H mar 22, 2022 | 720.30H 61.43M | 5153.20H | -220.30H |

| AD's |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | AD No. | Description | Type | Perf. | TTSC | Due | Rem. |
| OY-BUF | AD 2011-10-09 Tolerance ±10% | Seat track inspection ( superseeds 87-20-03R3). | Recur | 4869.28H apr 29, 2026 | 504.22H 12.20M | 4969.28H apr 29, 2027 | -404.22H -0.20M |
| OY-BUF | AD 2026-04-11 Engine | Inspect engine oil for bronze metal particulates. | Recur | 4869.28H apr 29, 2026 | 504.22H 12.20M | 4919.28H apr 29, 2027 | -454.22H -0.20M |
| OY-BUF | AIR-21-16 | bolts that attach the seat back to the seat base of the pilot and copilot seats. | Recur | 4869.28H apr 29, 2026 | 504.22H 12.20M | 5069.28H | -304.22H |
| OY-BUF | EAD 2006-0375 | electrical switch insp. "carling" type. | Recur | 4869.28H apr 29, 2026 | 504.22H 12.20M | 4969.28H apr 29, 2027 | -404.22H -0.20M |
| OY-BUF | EASA SIB 2011-15R1 AIRFRAME GROUP Tolerance ±10% | Mode S & C transponder system - ground testing. | Recur | 4820.30H apr 3, 2025 | 553.20H 25.07M | apr 3, 2027 | -1.07M |
| OY-BUF | LYC SB 366C Engine | Carburettor throttle body screw insp.. | Recur | 4869.28H apr 29, 2026 | 504.22H 12.20M | 4919.28H apr 29, 2027 | -454.22H -0.20M |
| OY-BUF | SEB 01-03 R1 | Control yoke inspection. | Recur | 4820.30H apr 3, 2025 | 553.20H 25.07M | apr 3, 2026 | -13.07M |
| OY-BUF | SEB 02-02 | Flammable fluid lines insp. | Recur | 4869.28H apr 29, 2026 | 504.22H 12.20M | 4969.28H apr 29, 2027 | -404.22H -0.20M |
| OY-BUF | SEB 05-02 | Fuselage skin & fuel step insp/modification. | Recur | 4869.28H apr 29, 2026 | 504.22H 12.20M | 4969.28H apr 29, 2027 | -404.22H -0.20M |
| OY-BUF | SEB 94-08 | Horizontal stabilizer forward spar insp/mod. | Recur | 4869.28H apr 29, 2026 | 504.22H 12.20M | 4969.28H apr 29, 2027 | -404.22H -0.20M |
| OY-BUF | SEB 95-03 R1 | Flap support inspection & roller washer install. | Recur | 4869.28H apr 29, 2026 | 504.22H 12.20M | 4969.28H apr 29, 2027 | -404.22H -0.20M |
| OY-BUF | SEB 99-18 R1 | Fuel quantity indicating system insp. | Recur | 4869.28H apr 29, 2026 | 504.22H 12.20M | 4969.28H apr 29, 2027 | -404.22H -0.20M |

| Inspections |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reg. | Name | Limit | Perf. | TTSC | Due | Rem. |
| OY-BUF | Insurance Certificate | 12.00M | 4867.90H dec 10, 2025 | 505.60H 16.83M | dec 31, 2026 | -4.13M |
| OY-BUF | SID operation 01 | 12.00M | 4869.28H apr 29, 2026 | 504.22H 12.20M | apr 29, 2027 | -0.20M |
| OY-BUF | SID operation 03 | 24.00M | 4820.30H apr 3, 2025 | 553.20H 25.07M | apr 3, 2027 | -1.07M |
| OY-BUF | ARC renewal | 12.00M | 4836.80H jun 26, 2025 | 536.70H 22.30M | jun 28, 2026 | -10.23M |
| OY-BUF | AMP review | 12.00M | 4836.80H jun 23, 2025 | 536.70H 22.40M | jun 28, 2026 | -10.23M |
| OY-BUF | SID operation 02 | 12.00M | 4869.28H apr 29, 2026 | 504.22H 12.20M | apr 29, 2027 | -0.20M |
| OY-BUF | 50 Hrs insp Tolerance ±10% | 50.00H 12.00M | 4869.28H apr 29, 2026 | 504.22H 12.20M | 4919.28H apr 29, 2027 | -454.22H -0.20M |
| OY-BUF | 100 Hrs insp Tolerance ±10% | 100.00H 12.00M | 4869.28H apr 29, 2026 | 504.22H 12.20M | 4969.28H apr 29, 2027 | -404.22H -0.20M |
| OY-BUF | Check for new dent and note it on the "bulekort" | 100.00H | 4869.28H apr 29, 2026 | 504.22H 12.20M | 4969.28H | -404.22H |
| OY-BUF | SID operation 25 | 100.00H 12.00M | 4869.28H apr 29, 2026 | 504.22H 12.20M | 4969.28H apr 29, 2027 | -404.22H -0.20M |
| OY-BUF | 200 Hrs insp Tolerance ±10% | 200.00H 12.00M | 4869.28H apr 29, 2026 | 504.22H 12.20M | 5069.28H apr 29, 2027 | -304.22H -0.20M |
| OY-BUF | SID operation 16 | 500.00H 60.00M | 4769.50H apr 16, 2024 | 604.00H 36.63M | 5269.50H apr 16, 2029 | -104.00H 23.37M |
| OY-BUF | SID operation 27 | 600.00H 12.00M | 4869.28H apr 29, 2026 | 504.22H 12.20M | 5469.28H apr 29, 2027 | 95.78H -0.20M |

| Defects |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | REG | # | ATA | Description | Correction | Date | Due Date | TTAF |
| Pilots Reported | OY-BUF |  |  | Throttle handle stiff at low power/idle range. |  | maj 4, 2026 |  | 4873.50 |

Forecast Cost

Component list. REG; Component/Action; Occ.(Max); Part; Labour; Misc; Total. OY-BUF; CO Detector / Rep; 2.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; Induction Air Filter / Rep; 2.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; Instrument filter / Rep; 1.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; Sparkplugs / Rep; 1.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; Vacuum pump /; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; Total; 0.00$; 0.00$; 0.00$; 0.00$. Tasks List. REG; Task; Occ.(Max); Part; Labour; Misc; Total. OY-BUF; Annual inspection of ELT; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; Check of emergency equipment FIRST AID KIT, Extinguishers, Life Jacket; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; Engine - Valve Inspection - Remove rocker box covers and check for freedom; 1.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; Equipment Visual Inspection: A visual inspection of the equipment installed by this STC must be performed.; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; LH magneto. 500 hrs insp.; 1.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; Perform vacuum pump vane wear inspection.; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; RH magneto. 500 hrs insp.; 1.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; Total; 0.00$; 0.00$; 0.00$; 0.00$. AD's List. REG; AD's; Occ.(Max); Part; Labour; Misc; Total. OY-BUF; AD 2011-10-09; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; AD 2026-04-11; 10.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; AIR-21-16; 2.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; EAD 2006-0375; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; EASA SIB 2011-15R1; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; LYC SB 366C; 10.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; SEB 01-03 R1; 2.1; 0.00$; 8 032.50$; 0.00$; 8 032.50$. OY-BUF; SEB 02-02; 5.0; 0.00$; 11 475.00$; 0.00$; 11 475.00$. OY-BUF; SEB 05-02; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; SEB 94-08; 5.0; 0.00$; 1 912.50$; 0.00$; 1 912.50$. OY-BUF; SEB 95-03 R1; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; SEB 99-18 R1; 5.0; 0.00$; 53 550.00$; 0.00$; 53 550.00$. OY-BUF; Total; 0.00$; 74 970.00$; 0.00$; 74 970.00$. Inspection List. REG; Inspection; Occ.(Max); Part; Labour; Misc; Total. OY-BUF; Insurance Certificate; 1.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; SID operation 01; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; SID operation 03; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; ARC renewal; 1.9; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; AMP review; 1.9; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; SID operation 02; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; 50 Hrs insp; 10.1; 0.00$; 54 085.50$; 0.00$; 54 085.50$. OY-BUF; 100 Hrs insp; 5.0; 0.00$; 26 775.00$; 0.00$; 26 775.00$. OY-BUF; Check for new dent and note it on the "bulekort"; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; SID operation 25; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; 200 Hrs insp; 2.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; SID operation 16; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; SID operation 27; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-BUF; Total; 0.00$; 80 860.50$; 0.00$; 80 860.50$. Total. REG; Part; Labour; Misc; Total. OY-BUF; Total; 0.00$; 155 830.50$; 0.00$; 155 830.50$.

| This report has been prepared by: |  |  |
| --- | --- | --- |
| Signature | Authorization Number | Date |
| © Aircraft Maintenance Systems RD inc |  |  |