| Hangarvej B 2 DK-4000 Roskilde Danmark CVR:49667728. Ph. Office: +45 46 19 11 00; Fax:+45 46 19 11 15. Flyservice@aircat.dk. | Projection List OY-CAH | Actions. Cle - Cleaning; Ove - Overhaul. Rep - Replacement. Cal - Calibration; Ins - Inspection. |
| --- | --- | --- |

| Created from the following values |  |  |
| --- | --- | --- |
| Aircraft Time. Date; maj 5,2027. TTAF; 14 158.2. | Custom Cycle. Name; Total. Flight Segment; 0.0. Landings; 22 327.0. LDG; 0.0. Lifts; 0.0. | Meter data. Name; Time; Cycle. |

| Components |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | Components/Action | Serial No. | Limit | Perf. | TTSC | TTSN | Due | Remaining |
| OY-CAH | Carburetor / Rep Tolerance ±0% | 75054408 | 2000.00H 144.00M | 13390.90H apr 25, 2024 | 2171.90H 140.43M | 4574.20H 276.80M | 13986.30H aug 21, 2027 | -171.90H 3.53M |
| OY-CAH | Engine / Ove | L-35133-36A | 2000.00H 144.00M | 11985.00H jun 24, 2015 | 2173.20H 142.37M | 4575.50H 278.70M | 13985.00H jun 24, 2027 | -173.20H 1.63M |
| OY-CAH | Engine mount vibration isolator / Rep |  | 2000.00H 144.00M | 11985.00H jun 24, 2015 | 2173.20H 142.37M |  | 13985.00H jun 24, 2027 | -173.20H 1.63M |
| OY-CAH | FIRST AID KIT NIVEAU 2 (AOC & DIN 13157) / Rep | FH2-000000101 |  | 13389.60H feb 27, 2024 | 768.60H 38.27M |  | apr 30, 2027 | -0.17M |
| OY-CAH | FIRST AID KIT NIVEAU 3 (AOC) / Rep | FH3-000000158 |  | 13553.10H aug 20, 2025 | 605.10H 20.50M |  | apr 30, 2027 | -0.17M |
| OY-CAH | Flexible Hoses in engine compartment, Oil & Fuel / Rep |  | 2000.00H 144.00M | 11985.00H jun 24, 2015 | 2173.20H 142.37M |  | 13985.00H jun 24, 2027 | -173.20H 1.63M |
| OY-CAH | Fuel pump, mechanic / Ove Tolerance ±0% | 07MG17 | 2000.00H 144.00M | 11985.00H jun 24, 2015 | 2173.20H 142.37M |  | 13985.00H jun 24, 2027 | -173.20H 1.63M |
| OY-CAH | Induction Air Filter / Rep Tolerance ±10% |  | 100.00H 12.00M | 13594.80H nov 12, 2025 | 563.40H 17.77M |  | 13694.80H jul 31, 2028 | -463.40H 14.87M |
| OY-CAH | Inlet pressure filter / Rep |  | 2000.00H | 11985.20H jun 24, 2015 | 2173.00H 142.37M |  | 13985.20H | -173.00H |
| OY-CAH | Instr. CO detector / Rep |  | 12.00M | 13594.80H nov 12, 2025 | 563.40H 17.77M |  | nov 12, 2026 | -5.77M |
| OY-CAH | Instrument air filter / Rep Tolerance ±10% |  | 500.00H 12.00M | 13594.80H nov 12, 2025 | 563.40H 17.77M |  | 14094.80H nov 12, 2026 | -63.40H -5.77M |
| OY-CAH | Radiator Oil / Cle |  | 2000.00H 144.00M | 11985.00H jun 24, 2015 | 2173.20H 142.37M |  | 13985.00H jun 24, 2027 | -173.20H 1.63M |
| OY-CAH | Sparkplugs / Rep |  | 400.00H | 13607.10H jan 19, 2026 | 551.10H 15.53M |  | 14007.10H | -151.10H |
| OY-CAH | Vacuum pump / Ove Tolerance ±10% | A69188 | 72.00M | 13055.20H nov 12, 2020 | 1103.00H 77.77M |  | nov 12, 2026 | -5.77M |
| OY-CAH | Vacuum relief filter / Rep Tolerance ±10% |  | 100.00H 12.00M | 13594.80H nov 14, 2025 | 563.40H 17.70M |  | 13694.80H nov 14, 2026 | -463.40H -5.70M |

| Tasks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | Task # | Task | ATA Code | Action | Limit | Perf. | TTSC | Due | Rem. |
| OY-CAH | 14 Master | Annuel check of ELT | 2500 | Ins | 12.00M | 13594.80H nov 12, 2025 | 563.40H 17.77M | nov 12, 2026 | -5.77M |
| OY-CAH | 18 Master | Avionics check | 3400 | Cal | 12.00M | 13594.80H nov 12, 2025 | 563.40H 17.77M | nov 12, 2026 | -5.77M |
| OY-CAH Tolerance ±0% | 12 Master | Check of emergency equipment FIRST AID KIT, Extinguishers, Life Jacket | 2500 | Ins | 12.00M | 13594.80H nov 12, 2025 | 563.40H 17.77M | nov 12, 2026 | -5.77M |
| OY-CAH | 32 - CAMO | DEFERRED DEFECT inspection every 100 hrs. Perform inspection of Elevator aft trim drum. The trim drum is reported worn but with in limit. New part ordered. Long delevery time. IPC 27-30-03 ITEM 10... PN TB10 2709600000 |  | Ins | 100.00H | 13594.80H | 563.40H | 13694.80H | -463.40H |
| OY-CAH | 33 - CAMO | DEFERRED DEFECT inspection every 100 hrs. Rudder lower bearing reported worn. But with in limit Perform inspection for continius operartion . IPC 55-30-00 FIG 1 ITEM 067 PN 7747750006 |  |  | 100.00H | 13594.80H | 563.40H | 13694.80H | -463.40H |
| OY-CAH Tolerance ±0% | 3 | Dual magneto: perform 500 hrs insp IAW TCM SB 643 on component: Magneto DUAL Make:TCM Model:D4LN-3000 Part Number:10-682555-11 Serial Number:3558 Tracking #:3660 | 7400 | Ins | 500.00H | 13594.80H nov 12, 2025 | 563.40H 17.77M | 14094.80H | -63.40H |
| OY-CAH Tolerance ±0% | 30 Master | Engine mount inspection at engine remove. | 7120 | Ins | 2000.00H 144.00M | 11985.00H jun 24, 2015 | 2173.20H 142.37M | 13985.00H jun 24, 2027 | -173.20H 1.63M |
| OY-CAH | 17 Master | GPS no.1 Antenna Garmin Ga-35. Conduct a visual inspection on the antenna and its mounting. | 3400 | Ins | 12.00M | 13594.80H nov 12, 2025 | 563.40H 17.77M | nov 12, 2026 | -5.77M |
| OY-CAH Tolerance ±0% | 20 Master | GTN, Inspection of switches, wiring and harnesses. | 3400 | Ins | 12.00M | 13594.80H nov 12, 2025 | 563.40H 17.77M | nov 12, 2026 | -5.77M |
| OY-CAH Tolerance ±0% | 21 Master | GTX 33/330 and 3X5, visual inspection of the equipment installed | 2320 | Ins | 12.00M | 13594.80H nov 12, 2025 | 563.40H 17.77M | nov 12, 2026 | -5.77M |
| OY-CAH Tolerance ±0% | 04 Master | Inspect A/P servo actuator for setting and corrosion. | 2200 | ins | 1000.00H | 12966.25H feb 3, 2020 | 1191.95H 87.07M | 13966.25H | -191.95H |
| OY-CAH | 16 Master | Propeller dynamic balacing | 6100 | Cal | 12.00M | 13594.80H nov 12, 2025 | 563.40H 17.77M | nov 12, 2026 | -5.77M |
| OY-CAH Tolerance ±10% | 2 | Vacuum pump vane wear inspection. on component: Vacuum pump Make:Rapco Model:Wear insp port. Part Number:RAP215CC Serial Number:A69188 Tracking #:14520 | 3700 | Ins | 100.00H | 13594.80H nov 12, 2025 | 563.40H 17.77M | 13694.80H | -463.40H |
| OY-CAH Tolerance ±0% | 31 Master | Valve Inspection - Remove rocker box covers ref. to Special Service Publication No. SSP1776 | 7200 | Ins | 400.00H | 13553.10H aug 20, 2025 | 605.10H 20.50M | 13953.10H | -205.10H |
| OY-CAH Tolerance ±0% | 08 Master | Visual inspect the wing structure for corrosion | 5700 | Ins | 2000.00H 36.00M | 13389.60H feb 27, 2024 | 768.60H 38.27M | 15389.60H feb 27, 2027 | 1231.40H -2.27M |
| OY-CAH Tolerance ±0% | 05 Master | Visually inspect A/P servo actuator supports, levers and attachments | 2200 | ins | 1000.00H | 12966.25H feb 3, 2020 | 1191.95H 87.07M | 13966.25H | -191.95H |
| OY-CAH Tolerance ±0% | 03 Master | Visually inspect flap actuator attachment for cracks. | 2700 | ins | 500.00H | 13389.60H feb 27, 2024 | 768.60H 38.27M | 13889.60H | -268.60H |
| OY-CAH Tolerance ±0% | 07 Master | Visually inspect flap control linkage. | 2700 | ins | 1000.00H | 12966.25H feb 3, 2020 | 1191.95H 87.07M | 13966.25H | -191.95H |
| OY-CAH Tolerance ±0% | 06 Master | Visually inspect flight control supports at specific frames. | 5300 | ins | 500.00H | 13389.60H feb 27, 2024 | 768.60H 38.27M | 13889.60H | -268.60H |
| OY-CAH Tolerance ±0% | 13 Master | Weighing and Balance of aircraft. | 0800 | Cal | 48.00M | 13353.70H nov 8, 2022 | 804.50H 53.90M | nov 8, 2026 | -5.90M |
| OY-CAH | 12 | Whelen Landing and Taxi Light Inspection IAW. STC Manual Form No. 14793D - Perform function check. Check that all LEDs are illuminated - Inspect lens form abrasion, crazing or cracking. - Check mounting connections and wire integrity. | 3300 | Ins | 12.00M | 13594.80H nov 12, 2025 | 563.40H 17.77M | nov 12, 2026 | -5.77M |

| AD's |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | AD No. | Description | Type | Perf. | TTSC | Due | Rem. |
| OY-CAH | CDN 89-097 (A) rev 1 AIRFRAME GROUP | Safety belt (SB 10-044). | Recur | 13594.80H nov 12, 2025 | 563.40H 17.77M | nov 12, 2026 | -5.77M |
| OY-CAH | FAA 2026-04-11 Engine | perform a visual inspection of the engine oil filter, oil pressure screen, and oilsuction screen (depending on the engine configuration) for bronze metal particulates. | Recur | 13643.50H apr 10, 2026 | 514.70H 12.83M | 13698.50H | -459.70H |
| OY-CAH | Lyc SB 366 | Carburetor Throttle Body Screw Inspection. Carburettor G-55-11749 | Recur | 13643.50H apr 10, 2026 | 514.70H 12.83M | 13693.50H | -464.70H |
| OY-CAH | SB 10-058-52 rev 2 | Cabin access door hooks. | Recur | 10106.00H maj 3, 2007 | 4052.20H 240.07M | 13694.80H nov 12, 2026 | -463.40H -5.77M |
| OY-CAH | SB 10-114-55 AIRFRAME GROUP | Rudder bearing 12 mnd insp (CDN 2001-002 (A)). | Recur | 13594.80H nov 12, 2025 | 563.40H 17.77M | nov 12, 2026 | -5.77M |
| OY-CAH | SB 10-142-25 AIRFRAME GROUP | Front seat backs. | Recur | 13643.50H apr 10, 2026 | 514.70H 12.83M | 13693.50H | -464.70H |
| OY-CAH | TCM SB 645 | Impulse coupling inspection. | Recur | 13594.80H nov 12, 2025 | 563.40H 17.77M | 14094.80H | -63.40H |

| Inspections |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reg. | Name | Limit | Perf. | TTSC | Due | Rem. |
| OY-CAH | Annual Inspection Tolerance ±0% | 12.00M | 13594.80H nov 12, 2025 | 563.40H 17.77M | nov 12, 2026 | -5.77M |
| OY-CAH | Insurance Certificate | 12.00M | 13602.30H dec 18, 2025 | 555.90H 16.57M | dec 31, 2026 | -4.13M |
| OY-CAH | ARC renewal | 12.00M | 13526.75H jun 26, 2025 | 631.45H 22.30M | jun 27, 2026 | -10.27M |
| OY-CAH | AMP review | 12.00M | 13526.75H jun 26, 2025 | 631.45H 22.30M | jun 27, 2026 | -10.27M |
| OY-CAH | 50 Hour Inspection - CAT Practice | 50.00H | 13643.50H apr 10, 2026 | 514.70H 12.83M | 13693.50H | -464.70H |
| OY-CAH | 50 hrs Inspection Tolerance ±0% | 50.00H 6.00M | 13643.50H apr 10, 2026 | 514.70H 12.83M | 13693.50H okt 10, 2026 | -464.70H -6.83M |
| OY-CAH | Check for new dents and note it on the "Bulekort" | 100.00H | 13594.80H nov 12, 2025 | 563.40H 17.77M | 13694.80H | -463.40H |
| OY-CAH | Cylinder pressure test | 100.00H | 13594.80H nov 12, 2025 | 563.40H 17.77M | 13694.80H | -463.40H |
| OY-CAH | 100 Hour Inspection Tolerance ±0% | 100.00H | 13594.80H nov 12, 2025 | 563.40H 17.77M | 13694.80H | -463.40H |
| OY-CAH | 2000 hrs / Major inspection | 2000.00H | 11985.00H jun 24, 2015 | 2173.20H 142.37M | 13985.00H | -173.20H |

| Defects |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | REG | # | ATA | Description | Correction | Date | Due Date | TTAF |
| Notes | OY-CAH | 1 | 2500 | {WR#7046 ITEM#64102}Pilot sidewall carpet worn, Deferred due to CAT approval.. Previous correction: Trensfer to Item 16. Close item. | Trensfer to Item 16. Close item. | aug 9, 2017 | jan 1, 2026 | 13490.00 |
| Notes | OY-CAH | 2 | 2500 | {WR#7046 ITEM#64101}Carpet in baggage compartment missing.. Previous correction: Trensfer to Item 16. Close item. | Trensfer to Item 16. Close item. | aug 9, 2017 | jan 1, 2026 | 13490.00 |
| Deferred | OY-CAH | 2 |  | {WR# 6013 ITEM#4}Rudder lower bearing worn... IPC 55-30-00 FIG 1 ITEM 067 PN 7747750006 1 EA ..... RIVET PN 5528340401 REQ 4770. 4 EA...(FHA) 08NOV2024. NO BALLJOINT AT STUCK 08NOV2024 (FHA). Previous correction: Due to parts - Ref. CAMO - DEFFERED.. Previous correction: Due to parts - Ref. CAMO - DEFFERED. | Due to parts - Ref. CAMO - DEFFERED. | nov 11, 2024 | apr 1, 2026 | 13477.60 |
| Notes | OY-CAH | 3 | 2500 | {WR#7046 ITEM#64099}CTR console from pilot and aft, carpet worn. Deferred due CAT approval. No operating stickers avaiable.. Previous correction: Trensfer to Item 16. Close item. | Trensfer to Item 16. Close item. | aug 9, 2017 | jan 1, 2026 | 13490.00 |
| Notes | OY-CAH | 4 | 2500 | {WR#7046 ITEM#64096}Vertical carpet in front of RH backseat loose/damaged.. Previous correction: Trensfer to Item 16. Close item. | Trensfer to Item 16. Close item. | aug 9, 2017 | jan 1, 2026 | 13490.00 |
| Deferred | OY-CAH | 4 |  | {WR# 6013 ITEM#4}Elevator aft trim drum worn. IPC 27-30-03 ITEM 10... PN TB10 2709600000 REQ4773. Previous correction: Due to parts - Ref. CAMO - DEFFERED.. Previous correction: Due to parts - Ref. CAMO - DEFFERED. | Due to parts - Ref. CAMO - DEFFERED. | nov 11, 2024 | apr 1, 2026 | 13477.60 |

Forecast Cost

Component list. REG; Component/Action; Occ.(Max); Part; Labour; Misc; Total. OY-CAH; Carburetor / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Engine / Ove; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Engine mount vibration isolator / Rep; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; FIRST AID KIT NIVEAU 2 (AOC & DIN 13157) / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; FIRST AID KIT NIVEAU 3 (AOC) / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Flexible Hoses in engine compartment, Oil & Fuel / Rep; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Fuel pump, mechanic / Ove; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Induction Air Filter / Rep; 5.6; 448.00$; 2 066.40$; 0.00$; 2 514.40$. OY-CAH; Inlet pressure filter / Rep; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Instr. CO detector / Rep; 1.5; 172.50$; 276.75$; 0.00$; 449.25$. OY-CAH; Instrument air filter / Rep; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Radiator Oil / Cle; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Sparkplugs / Rep; 1.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Vacuum pump / Ove; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Vacuum relief filter / Rep; 5.6; 560.00$; 2 066.40$; 0.00$; 2 626.40$. OY-CAH; Total; 1 180.50$; 4 409.55$; 0.00$; 5 590.05$. Tasks List. REG; Task; Occ.(Max); Part; Labour; Misc; Total. OY-CAH; Annuel check of ELT; 1.5; 0.00$; 553.50$; 0.00$; 553.50$. OY-CAH; Avionics check; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Check of emergency equipment FIRST AID KIT, Extinguishers, Life Jacket; 1.5; 0.00$; 1 107.00$; 0.00$; 1 107.00$. OY-CAH; DEFERRED DEFECT inspection every 100 hrs. Perform inspection of Elevator aft trim drum. The trim drum is reported worn but with in limit. New part ordered. Long delevery time. IPC 27-30-03 ITEM 10... PN TB10 2709600000; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; DEFERRED DEFECT inspection every 100 hrs. Rudder lower bearing reported worn. But with in limit Perform inspection for continius operartion . IPC 55-30-00 FIG 1 ITEM 067 PN 7747750006; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Dual magneto: perform 500 hrs insp IAW TCM SB 643; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Engine mount inspection at engine remove.; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; GPS no.1 Antenna Garmin Ga-35. Conduct a visual inspection on the antenna and its mounting.; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; GTN, Inspection of switches, wiring and harnesses.; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; GTX 33/330 and 3X5, visual inspection of the equipment installed; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Inspect A/P servo actuator for setting and corrosion.; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Propeller dynamic balacing; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Vacuum pump vane wear inspection.; 5.6; 0.00$; 4 132.80$; 0.00$; 4 132.80$. OY-CAH; Valve Inspection - Remove rocker box covers ref. to Special Service Publication No. SSP1776; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Visual inspect the wing structure for corrosion; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Visually inspect A/P servo actuator supports, levers and attachments; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Visually inspect flap actuator attachment for cracks.; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Visually inspect flap control linkage.; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Visually inspect flight control supports at specific frames.; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Weighing and Balance of aircraft.; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Whelen Landing and Taxi Light Inspection IAW. STC Manual Form No. 14793D - Perform function check. Check that all LEDs are illuminated - Inspect lens form abrasion, crazing or cracking. - Check mounting connections and wire integrity.; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Total; 0.00$; 5 793.30$; 0.00$; 5 793.30$. AD's List. REG; AD's; Occ.(Max); Part; Labour; Misc; Total. OY-CAH; CDN 89-097 (A) rev 1; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; FAA 2026-04-11; 9.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Lyc SB 366; 10.3; 0.00$; 1 900.35$; 0.00$; 1 900.35$. OY-CAH; SB 10-058-52 rev 2; 40.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; SB 10-114-55; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; SB 10-142-25; 10.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; TCM SB 645; 1.1; 0.00$; 1 217.70$; 0.00$; 1 217.70$. OY-CAH; Total; 0.00$; 3 118.05$; 0.00$; 3 118.05$. Inspection List. REG; Inspection; Occ.(Max); Part; Labour; Misc; Total. OY-CAH; Annual Inspection; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Insurance Certificate; 1.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; ARC renewal; 1.9; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; AMP review; 1.9; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; 50 Hour Inspection - CAT Practice; 10.3; 0.00$; 3 800.70$; 0.00$; 3 800.70$. OY-CAH; 50 hrs Inspection; 10.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Check for new dents and note it on the "Bulekort"; 5.6; 0.00$; 4 132.80$; 0.00$; 4 132.80$. OY-CAH; Cylinder pressure test; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; 100 Hour Inspection; 5.6; 0.00$; 33 062.40$; 0.00$; 33 062.40$. OY-CAH; 2000 hrs / Major inspection; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CAH; Total; 0.00$; 40 995.90$; 0.00$; 40 995.90$. Total. REG; Part; Labour; Misc; Total. OY-CAH; Total; 1 180.50$; 54 316.80$; 0.00$; 55 497.30$.

| This report has been prepared by: |  |  |
| --- | --- | --- |
| Signature | Authorization Number | Date |
| © Aircraft Maintenance Systems RD inc |  |  |