| Hangarvej B 2 DK-4000 Roskilde Danmark CVR:49667728. Ph. Office: +45 46 19 11 00; Fax:+45 46 19 11 15. Flyservice@aircat.dk. | Projection List OY-CDC | Actions. Ins - Inspection; On - On Condition. Ove - Overhaul; Rep - Replacement. Cal - Calibration. Ins - Inspection. Ser - Servicing. |
| --- | --- | --- |

| Created from the following values |  |  |
| --- | --- | --- |
| Aircraft Time. Date; maj 5,2027. TTAF; 6 355.4. | Custom Cycle. Name; Total. Flight Segment; 0.0. Landings; 9 777.0. Lifts; 0.0. | Meter data. Name; Time; Cycle. |

| Components |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | Components/Action | Serial No. | Limit | Perf. | TTSC | TTSN | Due | Remaining |
| OY-CDC | Air intake filter LH eng (L/H) / Rep Tolerance ±10% |  | 100.00H 12.00M | 5796.70H nov 21, 2025 | 558.74H 17.47M |  | 5896.70H jun 30, 2028 | -458.74H 13.83M |
| OY-CDC | Air intake filter RH eng (R/H) / Rep Tolerance ±10% |  | 100.00H 12.00M | 5796.70H nov 21, 2025 | 558.74H 17.47M |  | 5896.70H jun 30, 2028 | -458.74H 13.83M |
| OY-CDC | Antenna ADF / On << Not Installed >> |  |  |  |  |  | jan 13, 2026 | -15.73M |
| OY-CDC | Antenna COM1 / On << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-CDC | Antenna COM2 / On << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-CDC | Antenna DME / On << Not Installed >> |  |  |  |  |  | jan 13, 2026 | -15.73M |
| OY-CDC | Antenna Marker Beacon / On << Not Installed >> |  |  |  |  |  | jan 13, 2026 | -15.73M |
| OY-CDC | Antenna Transponder / On << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-CDC | Antenna UHF / On << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-CDC | Carbon Monxide Gas detector (CO) (Cockpit) / Rep |  | 12.00M | 5661.30H jul 18, 2025 | 694.14H 21.57M |  | jul 17, 2026 | -9.60M |
| OY-CDC | Central instrument air filter (L/H) / Rep |  | 500.00H 12.00M | 5661.30H jul 18, 2025 | 694.14H 21.57M |  | 6161.30H jul 18, 2026 | -194.14H -9.57M |
| OY-CDC | Central instrument air filter (R/H) / Rep |  | 500.00H 12.00M | 5661.30H jul 18, 2025 | 694.14H 21.57M |  | 6161.30H jul 18, 2026 | -194.14H -9.57M |
| OY-CDC | De-ice sytem / On << Not Installed >> |  |  |  |  |  | jan 14, 2026 | -15.70M |
| OY-CDC | ELT battery / Rep |  | 72.00M | 4526.00H dec 27, 2019 | 1829.44H 88.27M |  | jun 30, 2026 | -10.17M |
| OY-CDC | Engine (R/H) / Ove Tolerance ±0% | L-20647-51A | 2000.00H 144.00M | 5661.30H jul 18, 2025 | 1929.14H 115.47M | 7399.14H 213.66M | 6426.30H sep 21, 2029 | 70.86H 28.53M |
| OY-CDC | Engine flexible hoses RH (R/H) / Ove |  | 2000.00H 60.00M | 5661.30H jul 18, 2025 | 1829.44H 88.84M |  | 6526.00H nov 25, 2026 | 170.56H -5.33M |
| OY-CDC | Engine tubular mount (R/H) / Ove | LH OY-CDC | 2000.00H 144.00M | 5661.30H jul 18, 2025 | 1929.34H 115.47M | 3093.14H 193.41M | 6426.10H sep 21, 2029 | 70.66H 28.53M |
| OY-CDC | FIRST AID KIT NIVEAU 3 (AOC) / Rep | FH3-000000166 |  | 5725.60H okt 1, 2025 | 629.84H 19.13M |  | apr 30, 2027 | -0.17M |
| OY-CDC | Lifevest (cabin) / Ins Tolerance ±10% | UU0509265 | 12.00M | 4045.00H okt 20, 2016 | 2310.44H 126.50M |  | sep 29, 2026 | -7.20M |
| OY-CDC | Lifevest (cabin) / Ins Tolerance ±10% | LU0905321 | 12.00M | 4045.00H okt 20, 2016 | 2310.44H 126.50M |  | sep 29, 2026 | -7.20M |
| OY-CDC | Lifevest (cabin) / Ins Tolerance ±10% | UU0510115 | 12.00M | 4045.00H okt 20, 2016 | 2310.44H 126.50M |  | sep 29, 2026 | -7.20M |
| OY-CDC | Lifevest (cabin) / Ins Tolerance ±10% | UU0503235 | 12.00M | 4045.00H okt 20, 2016 | 2310.44H 126.50M |  | sep 29, 2026 | -7.20M |
| OY-CDC | Lifevest (cabin) / Ins Tolerance ±10% | UU0510179 | 12.00M | 4045.00H okt 20, 2016 | 2310.44H 126.50M |  | sep 29, 2026 | -7.20M |
| OY-CDC | Lifevest (cabin) / Ins Tolerance ±10% | UU0509343 | 12.00M | 4045.00H okt 20, 2016 | 2310.44H 126.50M |  | sep 29, 2026 | -7.20M |
| OY-CDC | Magneto LH pos (R/H) / Rep | 14081562 | 2000.00H | 5661.30H jul 18, 2025 | 2146.54H 21.57M |  | 6208.90H | -146.54H |
| OY-CDC | Nav, Strob and landing Light system / On << Not Installed >> |  |  |  |  |  | jan 14, 2026 | -15.70M |
| OY-CDC | PROP DE-ICE TIMER / Rep << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-CDC | PROP DEICE BRUSH BLOCK (L/H) / Rep << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-CDC | PROP DEICE BRUSH BLOCK (R/H) / Rep << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-CDC | Prop governor LH eng 1 (L/H) / Ove Tolerance ±10% | 1155799U | 2000.00H 144.00M | 5427.00H feb 16, 2024 | 2163.44H 114.16M | 4530.64H 238.77M | 6192.00H okt 30, 2029 | -163.44H 29.83M |
| OY-CDC | Propeller (L/H) / Ove | AU13076B | 2400.00H 72.00M | 5427.00H feb 16, 2024 | 1699.64H 77.53M | 5381.64H 149.19M | 7055.80H nov 19, 2026 | 700.36H -5.53M |
| OY-CDC | Sparkplugs LH eng (L/H) / Rep |  | 400.00H | 5796.70H nov 21, 2025 | 558.74H 17.47M |  | 6196.70H | -158.74H |
| OY-CDC | Sparkplugs RH eng (R/H) / Rep |  | 400.00H | 5661.30H jul 18, 2025 | 794.94H 32.24M |  | 5960.50H | -394.94H |
| OY-CDC | Stabilator trim actuator / Rep << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-CDC | Strobe light box / On << Not Installed >> |  |  |  |  |  | jan 14, 2026 | -15.70M |

| Tasks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | Task # | Task | ATA Code | Action | Limit | Perf. | TTSC | Due | Rem. |
| OY-CDC Tolerance ±10% | 1 | 500 hrs insp. LH engine LH mag. on component: Magneto LH pos (L/H) Make:SLICK/CHAMPION Model: Part Number:4373 Serial Number:14071403 Tracking #:7157 | 7400 | Ins | 500.00H | 5675.50H jul 29, 2025 | 679.94H 21.20M | 6175.50H | -179.94H |
| OY-CDC Tolerance ±10% | 2 | 500 hrs insp. LH engine RH mag. on component: Magneto RH pos (L/H) Make:Slick/Champion Model: Part Number:4370 Serial Number:L-01417 Tracking #:21754 | 7400 | Ins | 500.00H | 5427.00H feb 16, 2024 | 928.44H 38.63M | 5927.00H | -428.44H |
| OY-CDC Tolerance ±10% | 3 | 500 hrs insp. RH engine LH mag. on component: Magneto LH pos (R/H) Make:Slick/Champion Model: Part Number:4372 Serial Number:15020412 Tracking #:8063 | 7400 | Ins | 500.00H | 5171.40H mar 3, 2023 | 1184.04H 50.07M | 6132.30H | -223.14H |
| OY-CDC Tolerance ±10% | 4 | 500 hrs insp. RH engine RH mag. on component: Magneto RH pos (R/H) Make:Slick/Champion Model: Part Number:4370 Serial Number:F-01232 Tracking #:11489 | 7400 | Ins | 500.00H | 5609.80H nov 15, 2024 | 745.64H 29.67M | 6066.30H | -289.14H |
| OY-CDC | MASTER-16 | Annual inspection of ELT | 2560 | Ins | 12.00M | 5796.70H nov 21, 2025 | 558.74H 17.47M | nov 21, 2026 | -5.47M |
| OY-CDC | MASTER-19 | Antenna GA35 inspection ref. to Doc. ICA-GA35-01 ref. to item 4 & 5 | 3050 | Ins | 12.00M | 5796.70H nov 21, 2025 | 558.74H 17.47M | nov 21, 2026 | -5.47M |
| OY-CDC | MASTER-13 | Check of emergency equipment FIRST AID KIT, Extinguishers, Life Jacket | 2500 | Ins | 12.00M | 5661.30H jul 18, 2025 | 694.14H 21.57M | jul 18, 2026 | -9.57M |
| OY-CDC | MASTER-25 | De-ice boots: Add age 1 master | 3000 | Ser | 150.00H 6.00M | 5833.14H apr 10, 2026 | 522.30H 12.83M | 5983.14H okt 10, 2026 | -372.30H -6.83M |
| OY-CDC | MASTER-26 | De-ice boots: Add Icex II | 3000 | Ser | 50.00H | 5833.14H apr 10, 2026 | 522.30H 12.83M | 5883.14H | -472.30H |
| OY-CDC | MASTER-20 | GNS430W ICA inspection ref. to. ICA-1259 but only Doc. ICA-GNS430W-01 item 4 & 5 | 3400 | Ins | 12.00M | 5796.70H nov 21, 2025 | 558.74H 17.47M | nov 21, 2026 | -5.47M |
| OY-CDC Tolerance ±0% | MASTER-09 | LH Engine - Valve inspection ref. to AMM SPL A | 7100 | Ins | 400.00H | 5514.60H jul 19, 2024 | 840.84H 33.53M | 5914.60H | -440.84H |
| OY-CDC | MASTER-18 | MD-41 Annunciator ICA inspection ref. to ICA-MD41-( )-01 Item 4 & 5. The unit itself is on-condition | 3400 | Ins | 12.00M | 5796.70H nov 21, 2025 | 558.74H 17.47M | nov 21, 2026 | -5.47M |
| OY-CDC Tolerance ±10% | 15 | Perform wear inspection on LH Vacuum pump | 3700 | Ins | 100.00H | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5896.70H | -458.74H |
| OY-CDC Tolerance ±10% | 16 | Perform wear inspection on RH vacuum pump on component: Vacuum pump (R/H) Part Number:RAP441CC-23 Serial Number:C5428 Tracking #:19080 | 3700 | Ins | 100.00H | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5896.70H | -458.74H |
| OY-CDC Tolerance ±0% | MASTER-10 | RH Engine - Valve inspection ref. to AMM SPL A | 7100 | Ins | 400.00H | 5514.60H jul 19, 2024 | 840.84H 33.53M | 5914.60H | -440.84H |
| OY-CDC | MASTER-24 | Seat belt and shoulder harness check of contions | 2500 | Ins | 12.00M | 5796.70H nov 21, 2025 | 558.74H 17.47M | nov 21, 2026 | -5.47M |
| OY-CDC | MASTER-23 | Stand by Compass calibration | 3400 | Cal | 12.00M | 5833.14H apr 10, 2026 | 522.30H 12.83M | apr 10, 2027 | -0.83M |
| OY-CDC Tolerance ±0% | MASTER-14 | Weighing - perform weighing of aircraft | 0800 | Cal | 48.00M | 5159.80H feb 14, 2023 | 1195.64H 50.70M | feb 14, 2027 | -2.70M |
| OY-CDC | 12 | Wing Special inspection - Perform special insp on wing structure. on component: Wing structure, Airworthiness limitation Make:Partenavia/vulcan Air Serial Number:211 | 5700 | Ins | 6000.00H | 0.00H jan 1, 1980 | 6355.44H 568.13M | 6000.00H | -355.44H |

| AD's |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | AD No. | Description | Type | Perf. | TTSC | Due | Rem. |
| OY-CDC | AD 2015-19-07 Engine | Lycoming Engines fuel injected reciprocating engine models, R/H Engine. Engine (R/H) L-7782-51A | Recur | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5896.70H | -458.74H |
| OY-CDC | AD 2015-19-07 Engine | Lycoming Engines fuel injected reciprocating engine models, L/H Engine. Engine (L/H) | Recur | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5896.70H | -458.74H |
| OY-CDC | EAD 2021-0267 (PAD 21-158) Tolerance ±0% | RUDDER HINGES. | Recur | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5996.70H nov 21, 2026 | -358.74H -5.47M |
| OY-CDC | FAA AD 2024-21-02 Engine | Connecting Rod Assy Inspection. | Recur | 5833.14H apr 10, 2026 | 522.30H 12.83M | 5888.14H | -467.30H |
| OY-CDC | Lyc SB 388 C | Exhaust valve and guide inspection R/H. Engine R/H L-7782-51A | Recur | 5140.00H jan 31, 2023 | 1215.44H 51.13M | 6140.00H | -215.44H |
| OY-CDC | LYC SB 650 Engine | Intake Pipe and Flange Inspection. | Recur | 5833.14H apr 10, 2026 | 522.30H 12.83M | 5888.14H maj 10, 2027 | -467.30H 0.17M |
| OY-CDC | SB 113 AIRFRAME GROUP | FUEL SELECTOR CONTROL RIGGING (RAI AD 2002-415). | Recur | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5896.70H | -458.74H |
| OY-CDC | SB 67 REV 1 AIRFRAME GROUP | FIBRE GLASS LEADING EDGE. | Recur | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5896.70H | -458.74H |
| OY-CDC | SB 70 REV 2 AIRFRAME GROUP | ENG MOUNT/WING ATTACHMENT, section 3 inspection. | Recur | 5796.70H nov 21, 2025 | 558.74H 17.47M | 6296.70H nov 21, 2027 | -58.74H 6.53M |
| OY-CDC | SB 74 AIRFRAME GROUP | ELECTRICAL SYSTEM. | Recur | 5796.70H nov 21, 2025 | 558.74H 17.47M | 6296.70H nov 21, 2027 | -58.74H 6.53M |
| OY-CDC | SB 75 REV 3 AIRFRAME GROUP | FLIGHT CONTROL CABLES - 400 hrs insp. | Recur | 5609.80H nov 15, 2024 | 745.64H 29.67M | 6009.80H | -345.64H |
| OY-CDC | SB 75 REV 3 AIRFRAME GROUP | FLIGHT CONTROL CABLES - 100 hrs insp. | Recur | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5896.70H | -458.74H |
| OY-CDC | SB 83 Engine | EXHAUST MUFFLER ASSY. | Recur | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5996.70H | -358.74H |
| OY-CDC | SB 87 REV 1 AIRFRAME GROUP | STABILATOR TORQUE TUBE. | Recur | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5996.70H | -358.74H |
| OY-CDC | SB 91 AIRFRAME GROUP | FLIGHT CONTROL SHELVES INSPECTION. | Recur | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5996.70H nov 21, 2026 | -358.74H -5.47M |

| Inspections |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reg. | Name | Limit | Perf. | TTSC | Due | Rem. |
| OY-CDC | Insurance Certificate | 12.00M | 5801.70H dec 17, 2025 | 553.74H 16.60M | dec 31, 2026 | -4.13M |
| OY-CDC | AMP review | 12.00M | 5654.40H jul 7, 2025 | 701.04H 21.93M | jul 15, 2026 | -9.67M |
| OY-CDC | ARC Reneval | 12.00M | 5654.40H jul 7, 2025 | 701.04H 21.93M | jul 15, 2026 | -9.67M |
| OY-CDC | 50 hrs Inspection P68B & C Tolerance ±10% | 50.00H 12.00M | 5833.14H apr 10, 2026 | 522.30H 12.83M | 5883.14H apr 10, 2027 | -472.30H -0.83M |
| OY-CDC | CAT practice | 50.00H | 5833.14H apr 10, 2026 | 522.30H 12.83M | 5883.14H | -472.30H |
| OY-CDC | Cylinder pressure test | 100.00H | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5896.70H | -458.74H |
| OY-CDC | 100 hrs Inspection P68B & C Tolerance ±10% | 100.00H 12.00M | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5896.70H nov 21, 2026 | -458.74H -5.47M |
| OY-CDC | Check for new dent and note it on the "bulekort" | 100.00H | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5896.70H | -458.74H |
| OY-CDC | 100 hrs Lubrication P68C Tolerance ±0% | 100.00H 12.00M | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5896.70H nov 21, 2026 | -458.74H -5.47M |
| OY-CDC | 200 Hrs Lubrication P68C Tolerance ±0% | 200.00H 12.00M | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5996.70H nov 21, 2026 | -358.74H -5.47M |
| OY-CDC | 200 hrs inspection P68B & C Tolerance ±10% | 200.00H 12.00M | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5996.70H nov 21, 2026 | -358.74H -5.47M |
| OY-CDC | SPL H insp P68B & C Tolerance ±0% | 200.00H 12.00M | 5796.70H nov 21, 2025 | 558.74H 17.47M | 5996.70H nov 21, 2026 | -358.74H -5.47M |
| OY-CDC | SPL B insp P68B & C Tolerance ±0% | 500.00H 24.00M | 5796.70H nov 21, 2025 | 558.74H 17.47M | 6296.70H nov 21, 2027 | -58.74H 6.53M |
| OY-CDC | SPL F insp P68B & C Tolerance ±0% | 500.00H | 5514.60H jul 19, 2024 | 840.84H 33.53M | 6014.60H | -340.84H |
| OY-CDC | 500 Hrs Lubrication P68C Tolerance ±0% | 500.00H | 5514.60H jul 19, 2024 | 840.84H 33.53M | 6014.60H | -340.84H |
| OY-CDC | 1000 hrs Lubrication P68C Tolerance ±0% | 1000.00H | 5140.00H jan 31, 2023 | 1215.44H 51.13M | 6140.00H | -215.44H |

| Defects |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | REG | # | ATA | Description | Correction | Date | Due Date | TTAF |
| Pilots Reported | OY-CDC |  |  | Deviance between trim indicator and actual trim position. |  | apr 22, 2026 |  | 5844.25 |
| Deferred | OY-CDC | 10 | 2100 | Pilot Reported Defect. The ceiling and the wall directly behind and above the left front seat are extremely hot, even though the heating is not turned on or in use. I believe there is a heating hose running inside the wall, which may be damaged. | TROOPELSHOOTET IN AREA AND FOUNDT NO OPERTUNITIS OF WHAT COULD BE THE CAUSE OF THE HEATING IN LH WALL NEAR CAPTIN SEAT. NO HEAT WAS DETECTED DUING ENG RUN OP. | mar 8, 2026 |  | 5823.60 |
| Deferred | OY-CDC | 4 |  | Stall Warning U/S | Not performed | maj 6, 2025 | apr 1, 2026 | 5620.50 |
| Deferred | OY-CDC | 5 | 2200 | RH Eng Manifold Pressure Indication. Indication fluctuates between 15-20" and does not go any higher. |  | jan 12, 2026 | feb 28, 2026 | 5807.30 |
| Deferred | OY-CDC | 6 | 2300 | R/H Pilot Attitude Direction Indicator U/S |  | dec 10, 2025 | feb 28, 2026 | 5798.30 |
| Deferred | OY-CDC | 7 |  | Hot wall and ceiling |  | mar 6, 2026 | maj 15, 2026 | 5833.30 |
| Deferred | OY-CDC | 8 |  | Fuel pump light L/H engine |  | mar 5, 2026 | maj 15, 2026 | 5833.30 |
| Deferred | OY-CDC | 9 |  | L/H Engine, R/H Aft exhaust stack cracked at connection with R/H Fwd exhaust stack, needs replaced. Part ordered on Req# 5161 |  | apr 9, 2026 | maj 15, 2026 | 5833.30 |

Forecast Cost

Component list. REG; Component/Action; Occ.(Max); Part; Labour; Misc; Total. OY-CDC; Air intake filter LH eng (L/H) / Rep; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Air intake filter RH eng (R/H) / Rep; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Antenna ADF / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Antenna COM1 / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Antenna COM2 / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Antenna DME / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Antenna Marker Beacon / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Antenna Transponder / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Antenna UHF / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Carbon Monxide Gas detector (CO) (Cockpit) / Rep; 1.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Central instrument air filter (L/H) / Rep; 1.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Central instrument air filter (R/H) / Rep; 1.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; De-ice sytem / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; ELT battery / Rep; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Engine (R/H) / Ove; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Engine flexible hoses RH (R/H) / Ove; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Engine tubular mount (R/H) / Ove; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; FIRST AID KIT NIVEAU 3 (AOC) / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Lifevest (cabin) / Ins; 10.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Lifevest (cabin) / Ins; 10.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Lifevest (cabin) / Ins; 10.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Lifevest (cabin) / Ins; 10.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Lifevest (cabin) / Ins; 10.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Lifevest (cabin) / Ins; 10.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Magneto LH pos (R/H) / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Nav, Strob and landing Light system / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; PROP DE-ICE TIMER / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; PROP DEICE BRUSH BLOCK (L/H) / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; PROP DEICE BRUSH BLOCK (R/H) / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Prop governor LH eng 1 (L/H) / Ove; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Propeller (L/H) / Ove; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Sparkplugs LH eng (L/H) / Rep; 1.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Sparkplugs RH eng (R/H) / Rep; 1.7; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Stabilator trim actuator / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Strobe light box / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Total; 0.00$; 0.00$; 0.00$; 0.00$. Tasks List. REG; Task; Occ.(Max); Part; Labour; Misc; Total. OY-CDC; 500 hrs insp. LH engine LH mag.; 1.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; 500 hrs insp. LH engine RH mag.; 1.9; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; 500 hrs insp. RH engine LH mag.; 2.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; 500 hrs insp. RH engine RH mag.; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Annual inspection of ELT; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Antenna GA35 inspection ref. to Doc. ICA-GA35-01 ref. to item 4 & 5; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Check of emergency equipment FIRST AID KIT, Extinguishers, Life Jacket; 1.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; De-ice boots: Add age 1 master; 3.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; De-ice boots: Add Icex II; 10.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; GNS430W ICA inspection ref. to. ICA-1259 but only Doc. ICA-GNS430W-01 item 4 & 5; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; LH Engine - Valve inspection ref. to AMM SPL A; 2.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; MD-41 Annunciator ICA inspection ref. to ICA-MD41-( )-01 Item 4 & 5. The unit itself is on-condition; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Perform wear inspection on LH Vacuum pump; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Perform wear inspection on RH vacuum pump; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; RH Engine - Valve inspection ref. to AMM SPL A; 2.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Seat belt and shoulder harness check of contions; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Stand by Compass calibration; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Weighing - perform weighing of aircraft; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Wing Special inspection - Perform special insp on wing structure.; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Total; 0.00$; 0.00$; 0.00$; 0.00$. AD's List. REG; AD's; Occ.(Max); Part; Labour; Misc; Total. OY-CDC; AD 2015-19-07; 5.6; 0.00$; 2 066.40$; 0.00$; 2 066.40$. OY-CDC; AD 2015-19-07; 5.6; 0.00$; 2 066.40$; 0.00$; 2 066.40$. OY-CDC; EAD 2021-0267 (PAD 21-158); 2.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; FAA AD 2024-21-02; 9.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Lyc SB 388 C; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; LYC SB 650; 9.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; SB 113; 5.6; 0.00$; 20 580.00$; 0.00$; 20 580.00$. OY-CDC; SB 67 REV 1; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; SB 70 REV 2; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; SB 74; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; SB 75 REV 3; 1.9; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; SB 75 REV 3; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; SB 83; 2.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; SB 87 REV 1; 2.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; SB 91; 2.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; Total; 0.00$; 24 712.80$; 0.00$; 24 712.80$. Inspection List. REG; Inspection; Occ.(Max); Part; Labour; Misc; Total. OY-CDC; Insurance Certificate; 1.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; AMP review; 1.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; ARC Reneval; 1.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; 50 hrs Inspection P68B & C; 10.4; 44 886.40$; 153 504.00$; 0.00$; 198 390.40$. OY-CDC; CAT practice; 10.4; 0.00$; 23 025.60$; 0.00$; 23 025.60$. OY-CDC; Cylinder pressure test; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; 100 hrs Inspection P68B & C; 5.6; 0.00$; 45 460.80$; 0.00$; 45 460.80$. OY-CDC; Check for new dent and note it on the "bulekort"; 5.6; 0.00$; 4 132.80$; 0.00$; 4 132.80$. OY-CDC; 100 hrs Lubrication P68C; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDC; 200 Hrs Lubrication P68C; 2.8; 0.00$; 516.60$; 0.00$; 516.60$. OY-CDC; 200 hrs inspection P68B & C; 2.8; 0.00$; 16 531.20$; 0.00$; 16 531.20$. OY-CDC; SPL H insp P68B & C; 2.8; 0.00$; 5 166.00$; 0.00$; 5 166.00$. OY-CDC; SPL B insp P68B & C; 1.1; 0.00$; 2 841.30$; 0.00$; 2 841.30$. OY-CDC; SPL F insp P68B & C; 1.7; 0.00$; 3 763.80$; 0.00$; 3 763.80$. OY-CDC; 500 Hrs Lubrication P68C; 1.7; 0.00$; 3 136.50$; 0.00$; 3 136.50$. OY-CDC; 1000 hrs Lubrication P68C; 1.2; 0.00$; 221.40$; 0.00$; 221.40$. OY-CDC; Total; 44 886.40$; 258 300.00$; 0.00$; 303 186.40$. Total. REG; Part; Labour; Misc; Total. OY-CDC; Total; 44 886.40$; 283 012.80$; 0.00$; 327 899.20$.

| This report has been prepared by: |  |  |
| --- | --- | --- |
| Signature | Authorization Number | Date |
| © Aircraft Maintenance Systems RD inc |  |  |