| Hangarvej B 2 DK-4000 Roskilde Danmark CVR:49667728. Ph. Office: +45 46 19 11 00; Fax:+45 46 19 11 15. Flyservice@aircat.dk. | Projection List OY-CDJ | Actions. Cle - Cleaning; Ove - Overhaul. Rep - Replacement; Ret - Retire. Cal - Calibration. Cle - Cleaning. Ins - Inspection. |
| --- | --- | --- |

| Created from the following values |  |  |
| --- | --- | --- |
| Aircraft Time. Date; maj 5,2027. TTAF; 10 419.2. | Custom Cycle. Name; Total. Flight Segment; 0.0. Landings; 40 918.0. Lifts; 0.0. | Meter data. Name; Time; Cycle. |

| Components |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | Components/Action | Serial No. | Limit | Perf. | TTSC | TTSN | Due | Remaining |
| OY-CDJ | Carburetor / Ove Tolerance ±0% | BL2415632 | 2400.00H 172.00M | 9875.20H sep 5, 2025 | 2459.90H 117.76M | 2459.90H 117.76M | 10359.30H nov 12, 2031 | -59.90H 54.23M |
| OY-CDJ | Engine / Ove Tolerance ±0% | L-14697-39A | 2400.00H 172.00M | 7967.00H okt 31, 2017 | 2452.20H 114.13M | 10778.00H 114.13M | 10367.00H feb 29, 2032 | -52.20H 57.80M |
| OY-CDJ | Engine mount / Ret Tolerance ±0% | ??? | 10000.00H | 9879.80H okt 14, 2025 | 10419.20H 114.50M |  | 10000.00H | -419.20H |
| OY-CDJ | Engine mount vibration isolator / Rep Tolerance ±0% |  | 2400.00H 172.00M | 9879.80H okt 14, 2025 | 2452.20H 111.50M |  | 10367.00H maj 20, 2032 | -52.20H 60.50M |
| OY-CDJ | FIRST AID KIT NIVEAU 2 (AOC & DIN 13157) / Rep | FH2-000000084 |  | 9359.60H jul 11, 2023 | 1059.60H 45.80M |  | mar 31, 2027 | -1.13M |
| OY-CDJ | Fuel pump / Rep Tolerance ±0% | OB4J27 | 2400.00H 172.00M | 7967.00H okt 20, 2017 | 2452.20H 114.50M |  | 10367.00H feb 20, 2032 | -52.20H 57.50M |
| OY-CDJ | Fuel selector filter / Rep Tolerance ±10% |  | 300.00H | 9749.30H apr 7, 2025 | 669.90H 24.93M |  | 10049.30H | -369.90H |
| OY-CDJ | Induction Air Filter / Rep Tolerance ±0% |  | 100.00H 12.00M | 9888.90H mar 2, 2026 | 530.30H 14.10M |  | 9988.90H mar 2, 2027 | -430.30H -2.10M |
| OY-CDJ | Instr. CO detector (Cockpit) / Rep |  | 12.00M | 9888.90H feb 10, 2026 | 530.30H 14.83M |  | feb 10, 2027 | -2.83M |
| OY-CDJ | Instrument filter (Cockpit) / Rep Tolerance ±10% |  | 500.00H 12.00M | 9888.90H mar 2, 2026 | 530.30H 14.10M |  | 10388.90H mar 2, 2027 | -30.30H -2.10M |
| OY-CDJ | NLG mount / Ret Tolerance ±10% |  | 10000.00H | 0.00H jun 1, 1991 | 10419.20H 431.13M |  | 10000.00H | -419.20H |
| OY-CDJ | Propeller / Ove Tolerance ±0% | A63828 | 2000.00H | 7967.00H okt 20, 2017 | 2452.20H 114.50M |  | 9967.00H | -452.20H |
| OY-CDJ | Radiator Oil / Cle |  | 2400.00H 172.00M | 7967.00H okt 31, 2017 | 2452.20H 114.13M |  | 10367.00H feb 29, 2032 | -52.20H 57.80M |
| OY-CDJ | Spark plugs / Rep |  | 400.00H 0.00M | 9705.50H jan 31, 2025 | 713.70H 27.13M |  | 10105.50H | -313.70H |
| OY-CDJ | Vacuum relief filter / Rep Tolerance ±10% |  | 100.00H 12.00M | 9888.90H mar 2, 2026 | 530.30H 14.10M |  | 9988.90H mar 2, 2027 | -430.30H -2.10M |

| Tasks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | Task # | Task | ATA Code | Action | Limit | Perf. | TTSC | Due | Rem. |
| OY-CDJ Tolerance ±0% | 15 Master | Annuel check of ELT | 2500 | Ins | 12.00M | 9888.90H mar 2, 2026 | 530.30H 14.10M | mar 2, 2027 | -2.10M |
| OY-CDJ | 18 Master | Avionics check | 3400 | Cal | 12.00M | 9888.90H mar 2, 2026 | 530.30H 14.10M | mar 2, 2027 | -2.10M |
| OY-CDJ Tolerance ±0% | 12 Master | Check of emergency equipment FIRST AID KIT, Extinguishers, Life Jacket | 2500 | Ins | 12.00M | 9888.90H mar 2, 2026 | 530.30H 14.10M | mar 2, 2027 | -2.10M |
| OY-CDJ Tolerance ±10% | 11 Master | Cleaning of filter in fuel selector | 2820 | Cle | 100.00H | 9888.90H mar 2, 2026 | 530.30H 14.10M | 9988.90H | -430.30H |
| OY-CDJ Tolerance ±0% | 13 Master | Engine Mount, Inspection / Check at engine removal ref. AMM 71-20-00 | 7100 | Ins | 2400.00H 172.00M | 7967.00H okt 31, 2017 | 2452.20H 114.13M | 10367.00H feb 29, 2032 | -52.20H 57.80M |
| OY-CDJ Tolerance ±0% | 03 Master | LH magneto - 500 hrs insp. on component: Magneto (L/H) Make:SLICK/CHAMPION Part Number:4373 Serial Number:08062761 Tracking #:4380 | 7100 | Ins | 500.00H | 9875.20H sep 5, 2025 | 544.00H 20.00M | 10375.20H | -44.00H |
| OY-CDJ Tolerance ±10% | 06 Master | Perform visual inspection of flap actuator, support and attach fittings | 2750 | Ins | 500.00H | 9845.58H jul 31, 2025 | 573.62H 21.13M | 10345.58H | -73.62H |
| OY-CDJ Tolerance ±0% | 14 Master | Pitot static Test, Static pressure system, ref. FAR 91.411 | 3400 | Ins | 24.00M | 9656.90H okt 24, 2024 | 762.30H 30.37M | okt 24, 2026 | -6.37M |
| OY-CDJ | 17 Master | Propeller balanced | 6100 | Cal | 12.00M | 9888.90H mar 2, 2026 | 530.30H 14.10M | mar 2, 2027 | -2.10M |
| OY-CDJ Tolerance ±0% | 05 Master | RH magneto - 500 hrs insp. on component: Magneto (R/H) Make:SLICK Model:66GP-0SANN Part Number:66GP-0SANN Serial Number:21061045 Tracking #:18900 | 7100 | Ins | 500.00H | 9845.58H jul 31, 2025 | 573.62H 21.13M | 10345.58H | -73.62H |
| OY-CDJ Tolerance ±0% | 22 Master | Vacuum pump vane wear inspection. | 3700 | Ins | 100.00H 12.00M | 9888.90H mar 2, 2026 | 530.30H 14.10M | 9988.90H mar 2, 2027 | -430.30H -2.10M |
| OY-CDJ Tolerance ±0% | 23 Master | Valve Inspection - Remove rocker box covers and check for freedom of valve rockers ref. to Special Service Publication No. SSP1776. | 7200 | Ins | 400.00H | 9888.90H mar 2, 2026 | 530.30H 14.10M | 10288.90H | -130.30H |

| AD's |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | AD No. | Description | Type | Perf. | TTSC | Due | Rem. |
| OY-CDJ | AD 2025-0160 | Lower Rudder Hinge Fitting Inspection IAW SB 10-114-55. | Recur | 9845.58H jul 31, 2025 | 573.62H 21.13M | jul 31, 2026 | -9.13M |
| OY-CDJ | LYC SB 366 C Engine | Carburator Throttle Body Screw Inspection . Carburetor 75007602 | Recur | 9888.90H mar 2, 2026 | 530.30H 14.10M | 9938.90H | -480.30H |
| OY-CDJ | SB 10-058 R2 | Cabin access doors hooks. | Recur | 6380.00H maj 10, 2010 | 4039.20H 203.83M | 9945.58H jul 31, 2026 | -473.62H -9.13M |
| OY-CDJ | SB 10-142 AIRFRAME GROUP | Pilot + Co Pilot seat back insp. | Recur | 9888.90H mar 2, 2026 | 530.30H 14.10M | 9938.90H mar 2, 2027 | -480.30H -2.10M |
| OY-CDJ | SB2-19A | Mandatory inspection of impulse coupled Slick by Champion magnetos SB 2-19. Magneto (L/H) 15050310 | Recur | 9603.90H aug 11, 2024 | 815.30H 32.80M | 10003.90H | -415.30H |
| OY-CDJ | SIB 2011-15R3 | Mode S and Mode C Transponder Systems: Ground Testing. Transponder 43813 | Recur | 9656.90H okt 24, 2024 | 762.30H 30.37M | okt 24, 2026 | -6.37M |

| Inspections |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reg. | Name | Limit | Perf. | TTSC | Due | Rem. |
| OY-CDJ | Insurance Certificate | 12.00M | 9890.10H mar 3, 2026 | 529.10H 14.07M | dec 31, 2026 | -4.13M |
| OY-CDJ | ARC Full documented review | 36.00M | 9513.50H feb 29, 2024 | 905.70H 38.20M | mar 1, 2027 | -2.13M |
| OY-CDJ | ARC review extension | 12.00M | 9888.90H mar 1, 2026 | 530.30H 14.13M | mar 1, 2027 | -2.13M |
| OY-CDJ | AMP review | 12.00M | 9888.90H mar 1, 2026 | 530.30H 14.13M | mar 1, 2027 | -2.13M |
| OY-CDJ | Annual inspection (AI) TB9 AMM (P / N Z00. 18010309E0R19) Tolerance ±16% | 12.00M | 9888.90H mar 2, 2026 | 530.30H 14.10M | mar 2, 2027 | -2.10M |
| OY-CDJ | 50 Hrs Inspection TB9 AMM (P / N Z00. 18010309E0R19) Tolerance ±10% | 50.00H 6.00M | 9888.90H mar 2, 2026 | 530.30H 14.10M | 9938.90H sep 2, 2026 | -480.30H -8.10M |
| OY-CDJ | 50 hrs inspection add tasks for CAT practice Tolerance ±10% | 50.00H | 9888.90H mar 2, 2026 | 530.30H 14.10M | 9938.90H | -480.30H |
| OY-CDJ | Check for new dent and note it on the "Bulekort" | 100.00H | 9888.90H mar 2, 2026 | 530.30H 14.10M | 9988.90H | -430.30H |
| OY-CDJ | Cylinder pressure test Tolerance ±0% | 100.00H | 9888.90H mar 2, 2026 | 530.30H 14.10M | 9988.90H | -430.30H |
| OY-CDJ | 100 Hrs Inspection TB9 AMM (P / N Z00. 18010309E0R19) Tolerance ±10% | 100.00H | 9888.90H mar 2, 2026 | 530.30H 14.10M | 9988.90H | -430.30H |

| Defects |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | REG | # | ATA | Description | Correction | Date | Due Date | TTAF |
| Deferred | OY-CDJ | 1 | 3200 | L/H + R/H Brake Calipers worn. |  | mar 2, 2026 | jun 30, 2026 | 9888.90 |
| Deferred | OY-CDJ | 2 |  | RH REAR BULKHEAD ASSY P/N:TB09 5401200901 FOUNDT WOORN AND NEEDS REPLACEMENT. | PART NO IN STOCK. PART HAVE BEEN ORDRED REQ NR. 5082 IPC: 71-10-02 06A ITEM 010A (JRW) -----------POSTPONED-------------- IN AGREEMENT WITH CAMO. | mar 2, 2026 | jun 30, 2026 | 9888.90 |
| Deferred | OY-CDJ | 3 |  | BAFFLE SPRINGS ON LH & RH SIDE OF ENGINE MISSING. INSTALL NEW SPRING IPC: 71-10-02 03A (LH SIDE) ITEM: 260 P/N: TB10 09055100 IPC: 71-10-02 06A (RH SIDE) ITEM: 190 P/N: TB10 09055100 | PARTS NOT IN STOCK. PARTS HAVE BEEN ORDRED (D.9/2-2026) WITH REQ. NR. 5086 (JRW) MAIL ACORDING TO PARTS ADDED UNDER THIS ITEM -----------POSTPONED-------------- IN AGREEMENT WITH CAMO. | okt 24, 2025 | jun 30, 2026 | 9888.90 |

Forecast Cost

Component list. REG; Component/Action; Occ.(Max); Part; Labour; Misc; Total. OY-CDJ; Carburetor / Ove; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Engine / Ove; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Engine mount / Ret; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Engine mount vibration isolator / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; FIRST AID KIT NIVEAU 2 (AOC & DIN 13157) / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Fuel pump / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Fuel selector filter / Rep; 2.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Induction Air Filter / Rep; 5.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Instr. CO detector (Cockpit) / Rep; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Instrument filter (Cockpit) / Rep; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; NLG mount / Ret; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Propeller / Ove; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Radiator Oil / Cle; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Spark plugs / Rep; 1.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Vacuum relief filter / Rep; 5.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Total; 0.00$; 0.00$; 0.00$; 0.00$. Tasks List. REG; Task; Occ.(Max); Part; Labour; Misc; Total. OY-CDJ; Annuel check of ELT; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Avionics check; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Check of emergency equipment FIRST AID KIT, Extinguishers, Life Jacket; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Cleaning of filter in fuel selector; 5.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Engine Mount, Inspection / Check at engine removal ref. AMM 71-20-00; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; LH magneto - 500 hrs insp.; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Perform visual inspection of flap actuator, support and attach fittings; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Pitot static Test, Static pressure system, ref. FAR 91.411; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Propeller balanced; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; RH magneto - 500 hrs insp.; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Vacuum pump vane wear inspection.; 5.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Valve Inspection - Remove rocker box covers and check for freedom of valve rockers ref. to Special Service Publication No. SSP1776.; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Total; 0.00$; 0.00$; 0.00$; 0.00$. AD's List. REG; AD's; Occ.(Max); Part; Labour; Misc; Total. OY-CDJ; AD 2025-0160; 1.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; LYC SB 366 C; 10.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; SB 10-058 R2; 40.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; SB 10-142; 10.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; SB2-19A; 2.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; SIB 2011-15R3; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Total; 0.00$; 0.00$; 0.00$; 0.00$. Inspection List. REG; Inspection; Occ.(Max); Part; Labour; Misc; Total. OY-CDJ; Insurance Certificate; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; ARC Full documented review; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; ARC review extension; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; AMP review; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Annual inspection (AI) TB9 AMM (P / N Z00. 18010309E0R19); 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; 50 Hrs Inspection TB9 AMM (P / N Z00. 18010309E0R19); 10.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; 50 hrs inspection add tasks for CAT practice; 10.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Check for new dent and note it on the "Bulekort"; 5.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Cylinder pressure test; 5.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; 100 Hrs Inspection TB9 AMM (P / N Z00. 18010309E0R19); 5.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDJ; Total; 0.00$; 0.00$; 0.00$; 0.00$. Total. REG; Part; Labour; Misc; Total. OY-CDJ; Total; 0.00$; 0.00$; 0.00$; 0.00$.

| This report has been prepared by: |  |  |
| --- | --- | --- |
| Signature | Authorization Number | Date |
| © Aircraft Maintenance Systems RD inc |  |  |