| Hangarvej B 2 DK-4000 Roskilde Danmark CVR:49667728. Ph. Office: +45 46 19 11 00; Fax:+45 46 19 11 15. Flyservice@aircat.dk. | Projection List OY-CDU | Actions. Ove - Overhaul; Rep - Replacement. Cal - Calibration. Cle - Cleaning. Ins - Inspection. |
| --- | --- | --- |

| Created from the following values |  |  |
| --- | --- | --- |
| Aircraft Time. Date; maj 5,2027. TTAF; 6 782.1. | Custom Cycle. Name; Total. Flight Segment; 0.0. Landings; 11 387.0. Lifts; 0.0. | Meter data. Name; Time; Cycle. |

| Components |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | Components/Action | Serial No. | Limit | Perf. | TTSC | TTSN | Due | Remaining |
| OY-CDU | Engine / Ove Tolerance ±0% | L-11182-39A | 2400.00H 172.00M | 5599.00H maj 20, 2021 | 2578.60H 127.63M | 11966.90H 547.63M | 6603.50H jan 15, 2031 | -178.60H 44.33M |
| OY-CDU | Engine mount vibration isolator / Rep Tolerance ±0% |  | 2400.00H 172.00M | 5599.00H maj 21, 2021 | 2581.60H 127.60M |  | 6600.50H jan 16, 2031 | -181.60H 44.37M |
| OY-CDU | Fuel pump / Rep Tolerance ±0% | 07RH26 | 2400.00H 172.00M | 5599.00H maj 21, 2021 0.0C | 2578.60H 71.47M 0.0C |  | 6603.50H sep 21, 2035 | -178.60H 100.53M |
| OY-CDU | Fuel selector filter / Rep Tolerance ±10% |  | 300.00H | 6123.70H apr 25, 2025 | 658.40H 24.33M |  | 6423.70H | -358.40H |
| OY-CDU | Induction Air Filter / Rep Tolerance ±0% |  | 100.00H 12.00M | 6218.60H sep 9, 2025 | 563.50H 19.87M |  | 6318.60H sep 9, 2026 | -463.50H -7.87M |
| OY-CDU | Instr. CO detector (Cockpit) / Rep |  | 12.00M | 6218.60H sep 5, 2025 | 563.50H 20.00M |  | sep 5, 2026 | -8.00M |
| OY-CDU | Instrument filter (Cockpit) / Rep Tolerance ±10% |  | 500.00H 12.00M | 6268.40H feb 23, 2026 | 513.70H 14.40M |  | 6768.40H feb 23, 2027 | -13.70H -2.40M |
| OY-CDU | Spark plugs / Rep |  | 400.00H 0.00M | 6123.70H apr 25, 2025 | 658.40H 24.33M |  | 6523.70H | -258.40H |
| OY-CDU | Vacuum relief filter / Rep Tolerance ±0% |  | 100.00H 12.00M | 6218.60H sep 9, 2025 | 563.50H 19.87M |  | 6318.60H sep 9, 2026 | -463.50H -7.87M |

| Tasks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | Task # | Task | ATA Code | Action | Limit | Perf. | TTSC | Due | Rem. |
| OY-CDU Tolerance ±0% | 15 Master | Annuel check of ELT | 2500 | Ins | 12.00M | 6268.40H feb 23, 2026 | 513.70H 14.40M | feb 23, 2027 | -2.40M |
| OY-CDU | 18 Master | Avionics check | 3400 | Cal | 12.00M | 6268.40H feb 23, 2026 | 513.70H 14.40M | feb 23, 2027 | -2.40M |
| OY-CDU Tolerance ±0% | 12 Master | Check of emergency equipment FIRST AID KIT, Extinguishers, Life Jacket | 2500 | Ins | 12.00M | 6268.40H feb 19, 2026 | 513.70H 14.53M | feb 19, 2027 | -2.53M |
| OY-CDU Tolerance ±10% | 11 Master | Cleaning of filter in fuel selector | 2820 | Cle | 100.00H | 6218.60H sep 9, 2025 | 563.50H 19.87M | 6318.60H | -463.50H |
| OY-CDU Tolerance ±0% | 03 Master | LH magneto - 500 hrs insp. | 7100 | Ins | 500.00H | 6249.10H nov 6, 2025 | 533.00H 17.97M | 6749.10H | -33.00H |
| OY-CDU Tolerance ±10% | 06 Master | Perform visual inspection of flap actuator, support and attach fittings | 2750 | Ins | 500.00H | 5939.70H feb 26, 2024 | 842.40H 38.30M | 6439.70H | -342.40H |
| OY-CDU Tolerance ±5% | 07 Master | Perform visual inspection of flap linkage | 2750 | Ins | 1000.00H | 5472.55H dec 28, 2018 | 1309.55H 100.23M | 6472.55H | -309.55H |
| OY-CDU | 17 Master | Propeller balanced | 6100 | Cal | 12.00M | 6172.00H jul 8, 2025 | 610.10H 21.90M | jul 8, 2026 | -9.90M |
| OY-CDU Tolerance ±0% | 05 Master | RH magneto - 500 hrs insp. | 7100 | Ins | 500.00H | 5939.70H feb 26, 2024 | 842.40H 38.30M | 6439.70H | -342.40H |
| OY-CDU | 19 Master | Spinner, Propeller, Inspection iaw. Duc Helices Propellers manual for TB | 6100 | Ins | 100.00H 12.00M | 6218.60H sep 9, 2025 | 563.50H 19.87M | 6318.60H sep 9, 2026 | -463.50H -7.87M |
| OY-CDU Tolerance ±0% | 22 Master | Vacuum pump vane wear inspection. | 3700 | Ins | 100.00H 12.00M | 6218.60H sep 9, 2025 | 563.50H 19.87M | 6318.60H sep 9, 2026 | -463.50H -7.87M |
| OY-CDU Tolerance ±0% | 23 Master | Valve Inspection - Remove rocker box covers and check for freedom of valve rockers ref. to Special Service Publication No. SSP1776. | 7200 | Ins | 400.00H | 5939.70H feb 26, 2024 | 842.40H 38.30M | 6339.70H | -442.40H |

| AD's |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | AD No. | Description | Type | Perf. | TTSC | Due | Rem. |
| OY-CDU | AD 2025-0160 | Lower Rudder Hinge Fitting Inspection IAW SB 10-114-55. | Recur | 6268.40H feb 23, 2026 | 513.70H 14.40M | feb 23, 2027 | -2.40M |
| OY-CDU | FAA AD 2024-21-02 Engine | Connecting Rod Assy Inspection. | Recur | 6268.40H jan 22, 2026 | 513.70H 15.43M | 6323.40H | -458.70H |
| OY-CDU | LYC 388 C | Exhaust Valve and Guide Condition. | Recur | 5482.10H jul 24, 2019 | 1300.00H 93.37M | 6482.10H | -300.00H |
| OY-CDU | LYC SB 366 C Engine | Carburator Throttle Body Screw Inspection . Engine L-11182-39A | Recur | 6268.40H jan 22, 2026 | 513.70H 15.43M | 6318.40H | -463.70H |
| OY-CDU | SB 10-044 rev 1 | Safety belts - old AD ref: french CDN 89-097 rev 1. | Recur | 6268.40H feb 23, 2026 | 513.70H 14.40M | feb 23, 2027 | -2.40M |
| OY-CDU | SB 10-058 R2 | Cabin access doors hooks. | Recur | 3807.17H apr 30, 2010 | 2974.93H 204.17M | 6318.60H aug 9, 2026 | -463.50H -8.87M |
| OY-CDU | SB 10-142 | Pilot + Co Pilot seat back insp. | Recur | 6268.40H jan 22, 2026 | 513.70H 15.43M | 6318.40H jan 22, 2027 | -463.70H -3.43M |

| Inspections |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reg. | Name | Limit | Perf. | TTSC | Due | Rem. |
| OY-CDU | Insurance Certificate | 12.00M | 6267.00H dec 31, 2025 | 515.10H 16.13M | dec 31, 2026 | -4.13M |
| OY-CDU | ARC Full documented review | 36.00M | 5939.70H apr 8, 2024 | 842.40H 36.90M | apr 8, 2027 | -0.90M |
| OY-CDU | ARC review extension | 12.00M | 6282.10H apr 8, 2026 | 500.00H 12.90M | apr 8, 2027 | -0.90M |
| OY-CDU | AMP review | 12.00M | 6282.10H apr 8, 2026 | 500.00H 12.90M | apr 8, 2027 | -0.90M |
| OY-CDU | Annual inspection (AI) TB9 AMM (P / N Z00. 18010309E0R19) Tolerance ±16% | 12.00M | 6282.10H apr 8, 2026 | 500.00H 12.90M | apr 8, 2027 | -0.90M |
| OY-CDU | 50 Hrs Inspection TB9 AMM (P / N Z00. 18010309E0R19) Tolerance ±10% | 50.00H 6.00M | 6268.40H jan 22, 2026 | 513.70H 15.43M | 6318.40H jul 22, 2026 | -463.70H -9.43M |
| OY-CDU | 50 hrs inspection add tasks for CAT practice Tolerance ±10% | 50.00H | 6268.40H jan 22, 2026 | 513.70H 15.43M | 6318.40H | -463.70H |
| OY-CDU | Check for new dent and note it on the "Bulekort" | 100.00H | 6218.60H sep 9, 2025 | 563.50H 19.87M | 6318.60H | -463.50H |
| OY-CDU | Cylinder pressure test Tolerance ±0% | 100.00H | 6218.60H sep 9, 2025 | 563.50H 19.87M | 6318.60H | -463.50H |
| OY-CDU | 100 Hrs Inspection TB9 AMM (P / N Z00. 18010309E0R19) Tolerance ±10% | 100.00H | 6218.60H sep 9, 2025 | 563.50H 19.87M | 6318.60H | -463.50H |

Forecast Cost

Component list. REG; Component/Action; Occ.(Max); Part; Labour; Misc; Total. OY-CDU; Engine / Ove; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Engine mount vibration isolator / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Fuel pump / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Fuel selector filter / Rep; 2.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Induction Air Filter / Rep; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Instr. CO detector (Cockpit) / Rep; 1.7; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Instrument filter (Cockpit) / Rep; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Spark plugs / Rep; 1.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Vacuum relief filter / Rep; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Total; 0.00$; 0.00$; 0.00$; 0.00$. Tasks List. REG; Task; Occ.(Max); Part; Labour; Misc; Total. OY-CDU; Annuel check of ELT; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Avionics check; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Check of emergency equipment FIRST AID KIT, Extinguishers, Life Jacket; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Cleaning of filter in fuel selector; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; LH magneto - 500 hrs insp.; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Perform visual inspection of flap actuator, support and attach fittings; 1.7; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Perform visual inspection of flap linkage; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Propeller balanced; 1.8; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; RH magneto - 500 hrs insp.; 1.7; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Spinner, Propeller, Inspection iaw. Duc Helices Propellers manual for TB; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Vacuum pump vane wear inspection.; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Valve Inspection - Remove rocker box covers and check for freedom of valve rockers ref. to Special Service Publication No. SSP1776.; 2.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Total; 0.00$; 0.00$; 0.00$; 0.00$. AD's List. REG; AD's; Occ.(Max); Part; Labour; Misc; Total. OY-CDU; AD 2025-0160; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; FAA AD 2024-21-02; 9.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; LYC 388 C; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; LYC SB 366 C; 10.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; SB 10-044 rev 1; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; SB 10-058 R2; 29.7; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; SB 10-142; 10.3; 0.00$; 41.20$; 0.00$; 41.20$. OY-CDU; Total; 0.00$; 41.20$; 0.00$; 41.20$. Inspection List. REG; Inspection; Occ.(Max); Part; Labour; Misc; Total. OY-CDU; Insurance Certificate; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; ARC Full documented review; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; ARC review extension; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; AMP review; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Annual inspection (AI) TB9 AMM (P / N Z00. 18010309E0R19); 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; 50 Hrs Inspection TB9 AMM (P / N Z00. 18010309E0R19); 10.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; 50 hrs inspection add tasks for CAT practice; 10.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Check for new dent and note it on the "Bulekort"; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Cylinder pressure test; 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; 100 Hrs Inspection TB9 AMM (P / N Z00. 18010309E0R19); 5.6; 0.00$; 0.00$; 0.00$; 0.00$. OY-CDU; Total; 0.00$; 0.00$; 0.00$; 0.00$. Total. REG; Part; Labour; Misc; Total. OY-CDU; Total; 0.00$; 41.20$; 0.00$; 41.20$.

| This report has been prepared by: |  |  |
| --- | --- | --- |
| Signature | Authorization Number | Date |
| © Aircraft Maintenance Systems RD inc |  |  |