| Hangarvej B 2 DK-4000 Roskilde Danmark CVR:49667728. Ph. Office: +45 46 19 11 00; Fax:+45 46 19 11 15. Flyservice@aircat.dk. | Projection List OY-HHG | Actions. Rep - Replacement. Cal - Calibration; Cle - Cleaning. Ins - Inspection; Lub - Lubrication. Tes - Test. |
| --- | --- | --- |

| Created from the following values |  |  |
| --- | --- | --- |
| Aircraft Time. Date; maj 5,2027. TTAF; 2 952.3. | Custom Cycle. Name; Total. Flight Segment; 0.0. Landings; 7 382.0. Lifts; 0.0. | Meter data. Name; Time; Cycle. |

| Components |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | Components/Action | Serial No. | Limit | Perf. | TTSC | TTSN | Due | Remaining |
| OY-HHG | First Aid Kit Niveau 3 (AOC) / Rep | FH3-000000157 |  | 2339.80H aug 21, 2025 | 612.55H 20.47M |  | apr 30, 2027 | -0.17M |
| OY-HHG | Hydraulic Filter / Rep Tolerance ±0% |  | 300.00H | 2382.90H jan 14, 2026 | 569.45H 15.70M |  | 2682.90H | -269.45H |
| OY-HHG | Induction airfilter / Rep Tolerance ±0% |  | 100.00H 12.00M | 2382.90H jan 14, 2026 | 569.45H 15.70M |  | 2482.90H jan 14, 2027 | -469.45H -3.70M |
| OY-HHG | Lifevest 1 / Rep | UU0503248 | 120.00M | 1264.00H nov 8, 2018 | 1688.35H 101.90M |  | dec 26, 2026 | -4.30M |
| OY-HHG | Lifevest 2 / Rep | LU0902953 | 120.00M | 1264.00H nov 8, 2018 | 1688.35H 101.90M |  | feb 28, 2027 | -2.23M |
| OY-HHG | Lifevest 3 / Rep | UU0606260 | 120.00M | 1264.00H nov 8, 2018 | 1688.35H 101.90M |  | dec 26, 2026 | -4.30M |
| OY-HHG | Lifevest 4 / Rep | UU0514833 | 120.00M | 1264.00H nov 8, 2018 | 1688.35H 101.90M |  | feb 28, 2027 | -2.23M |

| Tasks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | Task # | Task | ATA Code | Action | Limit | Perf. | TTSC | Due | Rem. |
| OY-HHG Tolerance ±0% | Master 11 | Annual check of ELT. | 2560 | Ins | 12.00M | 2382.90H jan 14, 2026 | 569.45H 15.70M | jan 14, 2027 | -3.70M |
| OY-HHG Tolerance ±0% | Master 12 | Check of emergency equipment FIRST AID KIT, Extinguishers, Life Jacket | 2500 | Ins | 12.00M | 2382.90H jan 14, 2026 | 569.45H 15.70M | jan 14, 2027 | -3.70M |
| OY-HHG Tolerance ±0% | Master 08 | Drain and flush main rotor gearbox. | 6300 | Lub | 500.00H | 1999.80H sep 20, 2023 | 952.55H 43.50M | 2499.80H | -452.55H |
| OY-HHG Tolerance ±0% | Master 09 | Drain and flush tail rotor gearbox. | 6500 | Lub | 500.00H | 1999.80H sep 20, 2023 | 952.55H 43.50M | 2499.80H | -452.55H |
| OY-HHG Cycle Name : Flight Segment Tolerance ±0% | Master 01 | Float inspection LH | 2500 | Ins | 12.00M | 2382.90H jan 14, 2026 0.0C | 569.45H 15.70M 0.0C | jan 14, 2027 | -3.70M |
| OY-HHG Tolerance ±0% | Master 02 | Float inspection RH. | 2500 | Ins | 12.00M | 2382.90H jan 14, 2026 | 569.45H 15.70M | jan 14, 2027 | -3.70M |
| OY-HHG Tolerance ±0% | Master 04 | LH Magneto 500 hrs insp | 7400 | Ins | 500.00H | 2353.20H sep 12, 2025 | 599.15H 19.77M | 2853.20H | -99.15H |
| OY-HHG Tolerance ±0% | Master 03 | Lubricate C181-3 bearing. | 6300 | Lub | 300.00H 36.00M | 2382.90H jan 14, 2026 | 569.45H 15.70M | 2682.90H jan 14, 2029 | -269.45H 20.30M |
| OY-HHG Tolerance ±0% | Master 17 | Magnetic Compass Compensation | 3400 | Cal | 12.00M | 2382.90H jan 14, 2026 | 569.45H 15.70M | jan 14, 2027 | -3.70M |
| OY-HHG Tolerance ±0% | Master 18 | Night VFR life saver battery (Emergency), CAP test of Battery P/N 9015607 | 2400 | Tes | 12.00M | 2382.90H jan 14, 2026 | 569.45H 15.70M | jan 14, 2027 | -3.70M |
| OY-HHG Tolerance ±0% | Master 14 | Perform cleaning of gearbox chip detector per § 1.115 | 6340 | Cle | 500.00H 12.00M | 2439.60H apr 23, 2026 | 512.75H 12.40M | 2939.60H apr 23, 2027 | -12.75H -0.40M |
| OY-HHG Tolerance ±0% | Master 05 | RH Magneto 500 hrs insp. | 7400 | Ins | 500.00H | 2207.30H mar 6, 2025 | 745.05H 25.97M | 2707.30H | -245.05H |
| OY-HHG Tolerance ±0% | Master 13 | Storage max 120 days if kept dry/clear of direct sun. See AMP or CAT CAME sec. 1.16.3 (run engine at least 1 hrs (in flight) within 120 days to avoid this task | 7100 | Tes | 4.00M | 2382.10H jan 20, 2026 | 570.25H 15.50M | maj 20, 2026 | -11.50M |
| OY-HHG | Master 19 | Valve Inspection - Remove rocker box covers and check for freedom of valve rockers | 7200 | Ins | 300.00H | 2247.90H apr 30, 2025 | 704.45H 24.17M | 2547.90H | -404.45H |

| AD's |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | AD No. | Description | Type | Perf. | TTSC | Due | Rem. |
| OY-HHG | AD 2015-19-07 Engine | Lycoming Engines fuel injected reciprocating engine models. | Recur | 2382.90H jan 14, 2026 | 569.45H 15.70M | 2482.90H jan 14, 2027 | -469.45H -3.70M |
| OY-HHG | EAD 2005-0023 R3 | Exhaust valve guide clearence. | Recur | 2247.90H apr 30, 2025 | 704.45H 24.17M | 2547.90H | -404.45H |
| OY-HHG | LYC SB 650 Engine | Intake Pipe and Flange Inspection. | Recur | 2439.60H apr 23, 2026 | 512.75H 12.40M | 2494.60H maj 23, 2027 | -457.75H 0.60M |
| OY-HHG | Lyc. SB 301 | SERVICE LIMITATIONS FOR VALVES. | Recur | 2247.90H apr 30, 2025 | 704.45H 24.17M | 2547.90H | -404.45H |
| OY-HHG | SL-79 | Main Rotor Blade Unpainted Leading Edge Care. | Recur | 2382.90H jan 14, 2026 | 569.45H 15.70M | 2482.90H jan 14, 2027 | -469.45H -3.70M |
| OY-HHG | SL-81 | ELT Security. | Recur | 2382.90H jan 14, 2026 | 569.45H 15.70M | jan 14, 2027 | -3.70M |
| OY-HHG | SL-82A | Tail Rotor Blade Condition and Care. | Recur | 2382.90H jan 14, 2026 | 569.45H 15.70M | 2482.90H jan 14, 2027 | -469.45H -3.70M |
| OY-HHG | SL 45 AIRFRAME GROUP | Deleted Safety Wire, Maintenance personnel. | Recur | 2382.90H jan 14, 2026 | 569.45H 15.70M | 2482.90H jan 14, 2027 | -469.45H -3.70M |
| OY-HHG | SL 70 | Main rotor blade tip maintenance. | Recur | 2382.90H jan 14, 2026 | 569.45H 15.70M | 2482.90H jan 14, 2027 | -469.45H -3.70M |

| Inspections |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reg. | Name | Limit | Perf. | TTSC | Due | Rem. |
| OY-HHG | Insurance, 3 part | 12.00M | 2284.00H jun 18, 2025 | 668.35H 22.57M | jun 30, 2026 | -10.17M |
| OY-HHG | AMP review Tolerance ±0% | 12.00M | 2247.90H maj 6, 2025 | 704.45H 23.97M | maj 19, 2026 | -11.53M |
| OY-HHG | ARC reneval Tolerance ±0% | 12.00M | 2247.90H maj 6, 2025 | 704.45H 23.97M | maj 19, 2026 | -11.53M |
| OY-HHG | 50 Hrs inspection Tolerance ±10% | 50.00H 12.00M | 2439.60H apr 23, 2026 | 512.75H 12.40M | 2489.60H apr 23, 2027 | -462.75H -0.40M |
| OY-HHG | Check for new dent and note it on the "Bulekort" Tolerance ±0% | 100.00H | 2439.60H apr 23, 2026 | 512.75H 12.40M | 2539.60H | -412.75H |
| OY-HHG | 100 Hrs/Annual inspection Tolerance ±0% | 100.00H 12.00M | 2382.90H jan 14, 2026 | 569.45H 15.70M | 2482.90H jan 14, 2027 | -469.45H -3.70M |

Forecast Cost

Component list. REG; Component/Action; Occ.(Max); Part; Labour; Misc; Total. OY-HHG; First Aid Kit Niveau 3 (AOC) / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Hydraulic Filter / Rep; 1.9; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Induction airfilter / Rep; 5.7; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Lifevest 1 / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Lifevest 2 / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Lifevest 3 / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Lifevest 4 / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Total; 0.00$; 0.00$; 0.00$; 0.00$. Tasks List. REG; Task; Occ.(Max); Part; Labour; Misc; Total. OY-HHG; Annual check of ELT.; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Check of emergency equipment FIRST AID KIT, Extinguishers, Life Jacket; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Drain and flush main rotor gearbox.; 1.9; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Drain and flush tail rotor gearbox.; 1.9; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Float inspection LH; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Float inspection RH.; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; LH Magneto 500 hrs insp; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Lubricate C181-3 bearing.; 1.9; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Magnetic Compass Compensation; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Night VFR life saver battery (Emergency), CAP test of Battery P/N 9015607; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Perform cleaning of gearbox chip detector per § 1.115; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; RH Magneto 500 hrs insp.; 1.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Storage max 120 days if kept dry/clear of direct sun. See AMP or CAT CAME sec. 1.16.3 (run engine at least 1 hrs (in flight) within 120 days to avoid this task; 3.9; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Valve Inspection - Remove rocker box covers and check for freedom of valve rockers; 2.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Total; 0.00$; 0.00$; 0.00$; 0.00$. AD's List. REG; AD's; Occ.(Max); Part; Labour; Misc; Total. OY-HHG; AD 2015-19-07; 5.7; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; EAD 2005-0023 R3; 2.3; 0.00$; 9.20$; 0.00$; 9.20$. OY-HHG; LYC SB 650; 9.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Lyc. SB 301; 2.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; SL-79; 5.7; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; SL-81; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; SL-82A; 5.7; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; SL 45; 5.7; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; SL 70; 5.7; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; Total; 0.00$; 9.20$; 0.00$; 9.20$. Inspection List. REG; Inspection; Occ.(Max); Part; Labour; Misc; Total. OY-HHG; Insurance, 3 part; 1.9; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; AMP review; 2.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; ARC reneval; 2.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; 50 Hrs inspection; 10.3; 9 630.50$; 72.10$; 0.00$; 9 702.60$. OY-HHG; Check for new dent and note it on the "Bulekort"; 5.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-HHG; 100 Hrs/Annual inspection; 5.7; 0.00$; 79.80$; 0.00$; 79.80$. OY-HHG; Total; 9 630.50$; 151.90$; 0.00$; 9 782.40$. Total. REG; Part; Labour; Misc; Total. OY-HHG; Total; 9 630.50$; 161.10$; 0.00$; 9 791.60$.

| This report has been prepared by: |  |  |
| --- | --- | --- |
| Signature | Authorization Number | Date |
| © Aircraft Maintenance Systems RD inc |  |  |