| Hangarvej B 2 DK-4000 Roskilde Danmark CVR:49667728. Ph. Office: +45 46 19 11 00; Fax:+45 46 19 11 15. Flyservice@aircat.dk. | Projection List OY-OCM | Actions. Ins - Inspection; On - On Condition. Ove - Overhaul; Rep - Replacement. Cal - Calibration. Ins - Inspection. Ser - Servicing. |
| --- | --- | --- |

| Created from the following values |  |  |
| --- | --- | --- |
| Aircraft Time. Date; maj 5,2027. TTAF; 5 447.9. | Custom Cycle. Name; Total. Flight Segment; 0.0. Landings; 4 359.0. Lifts; 0.0. | Meter data. Name; Time; Cycle. |

| Components |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | Components/Action | Serial No. | Limit | Perf. | TTSC | TTSN | Due | Remaining |
| OY-OCM | Air intake filter (L/H) / Rep Tolerance ±10% |  | 100.00H 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M |  | 5047.90H feb 9, 2027 | -400.00H -2.87M |
| OY-OCM | Air intake filter (R/H) / Rep Tolerance ±10% |  | 100.00H 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M |  | 5047.90H feb 9, 2027 | -400.00H -2.87M |
| OY-OCM | Antenna ADF / On << Not Installed >> |  |  |  |  |  | jan 13, 2026 | -15.73M |
| OY-OCM | Antenna COM1 / On << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-OCM | Antenna COM2 / On << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-OCM | Antenna DME / On << Not Installed >> |  |  |  |  |  | jan 13, 2026 | -15.73M |
| OY-OCM | Antenna Marker Beacon / On << Not Installed >> |  |  |  |  |  | jan 13, 2026 | -15.73M |
| OY-OCM | Antenna Transponder / On << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-OCM | Antenna UHF / On << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-OCM | Carbon Monxide Gas detector (CO) (Cockpit) / Rep |  | 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M |  | feb 9, 2027 | -2.87M |
| OY-OCM | De-ice sytem / On << Not Installed >> |  |  |  |  |  | jan 14, 2026 | -15.70M |
| OY-OCM | Engine (L/H) / Ove Tolerance ±0% | L-19727-51A | 2200.00H 174.00M | 2997.38H maj 22, 2013 | 2450.52H 167.43M | 5447.92H 342.53M | 5197.38H nov 22, 2027 | -250.52H 6.57M |
| OY-OCM | Engine (R/H) / Ove Tolerance ±0% | L-19728-51A | 2200.00H 174.00M | 2997.36H maj 22, 2013 | 2450.54H 167.43M | 5447.94H | 5197.36H nov 22, 2027 | -250.54H 6.57M |
| OY-OCM | Engine tubular mount (L/H) / Ove Tolerance ±0% |  | 2200.00H 174.00M | 2997.38H maj 22, 2013 | 2450.52H 167.43M |  | 5197.38H nov 22, 2027 | -250.52H 6.57M |
| OY-OCM | Engine tubular mount (R/H) / Ove |  | 2200.00H 174.00M | 2997.38H maj 22, 2013 | 2450.52H 167.43M |  | 5197.38H nov 22, 2027 | -250.52H 6.57M |
| OY-OCM | FIRST AID KIT NIVEAU 3 (AOC) / Rep | FH3-000000150 |  | 4687.10H aug 2, 2024 | 760.80H 33.10M |  | aug 31, 2026 | -8.13M |
| OY-OCM | Fuel Injector (L/H) / Ove Tolerance ±0% | 12860-38 | 2200.00H 174.00M | 2997.38H maj 22, 2013 | 2450.52H 167.43M |  | 5197.38H nov 22, 2027 | -250.52H 6.57M |
| OY-OCM | Fuel Injector (R/H) / Ove | 51668 | 2200.00H 174.00M | 3088.90H aug 21, 2013 | 2359.00H 164.47M |  | 5288.90H feb 21, 2028 | -159.00H 9.53M |
| OY-OCM | Fuel Pump (L/H) (L/H) / Ove | 964505 | 2200.00H 174.00M | 2997.40H maj 22, 2013 | 2450.50H 167.43M |  | 5197.40H nov 22, 2027 | -250.50H 6.57M |
| OY-OCM | Fuel Pump (R/H) (R/H) / Ove | 964504 | 2200.00H 174.00M | 2997.40H maj 22, 2013 | 2450.50H 167.43M |  | 5197.40H nov 22, 2027 | -250.50H 6.57M |
| OY-OCM | Instrument airfilter / Rep |  | 500.00H 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M |  | 5447.90H feb 9, 2027 | 0.00H -2.87M |
| OY-OCM | Instrument airfilter (For A/P attitude ind) / Rep |  | 500.00H | 4731.10H aug 29, 2024 | 716.80H 32.20M |  | 5231.10H | -216.80H |
| OY-OCM | Magneto LH pos (R/H) / Rep | 11082201 | 2000.00H | 4870.20H aug 2, 2025 | 2381.80H 127.16M | 2381.80H 127.16M | 5066.10H | -381.80H |
| OY-OCM | Nav, Strob and landing Light system / On << Not Installed >> |  |  |  |  |  | jan 14, 2026 | -15.70M |
| OY-OCM | PROP DE-ICE TIMER / Rep << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-OCM | PROP DEICE BRUSH BLOCK (L/H) / Rep << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-OCM | PROP DEICE BRUSH BLOCK (R/H) / Rep << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-OCM | Rudder / << Not Installed >> |  |  |  |  |  | jan 9, 2026 | -15.87M |
| OY-OCM | Rudder trim actuator (Tail) / Rep << Not Installed >> |  |  |  |  |  | jan 9, 2026 | -15.87M |
| OY-OCM | Stabilator trim actuator / Rep << Not Installed >> |  |  |  |  |  | jan 12, 2026 | -15.77M |
| OY-OCM | Start Booster (L/H) / Ins Tolerance ±10% | 13055220 | 500.00H | 4872.60H aug 2, 2025 | 575.30H 21.10M |  | 5372.60H | -75.30H |
| OY-OCM | Start Booster (R/H) / Ins Tolerance ±10% | - | 500.00H | 4872.60H aug 2, 2025 | 575.30H 21.10M |  | 5372.60H | -75.30H |
| OY-OCM | Strobe light box / On << Not Installed >> |  |  |  |  |  | jan 14, 2026 | -15.70M |

| Tasks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | Task # | Task | ATA Code | Action | Limit | Perf. | TTSC | Due | Rem. |
| OY-OCM | 20 STC | Annual check of Garmin GNS 530W ref. to Gama Aviation ICA doc. DQ-14-263 last rev. (Issue 2) | 3400 | Ins | 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | feb 9, 2027 | -2.87M |
| OY-OCM | MASTER-16 | Annual inspection of ELT | 2560 | Ins | 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | feb 9, 2027 | -2.87M |
| OY-OCM | 21 STC | Antenna inspection. | 3050 | Ins | 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | feb 9, 2027 | -2.87M |
| OY-OCM | MASTER-13 | Check of emergency equipment FIRST AID KIT, Extinguishers, Life Jacket | 2500 | Ins | 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | feb 9, 2027 | -2.87M |
| OY-OCM | MASTER-25 | De-ice boots: Add age 1 master | 3000 | Ser | 150.00H 6.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5097.90H aug 31, 2026 | -350.00H -8.13M |
| OY-OCM | MASTER-26 | De-ice boots: Add Icex II | 3000 | Ser | 50.00H | 4947.90H feb 9, 2026 | 500.00H 14.87M | 4997.90H | -450.00H |
| OY-OCM Tolerance ±8% | MASTER-29 | L/H Engine - Engine internal corrosion inspection, TBO extension. | 7100 | Ins | 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | feb 9, 2027 | -2.87M |
| OY-OCM Tolerance ±0% | MASTER-27 | L/H Engine, Additional task (Inspection) on the engine when it is extended.(Inspection) | 7100 | Ins | 100.00H 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5047.90H feb 9, 2027 | -400.00H -2.87M |
| OY-OCM Tolerance ±0% | MASTER-09 | LH Engine - Valve inspection ref. to AMM SPL A | 7100 | Ins | 400.00H | 4872.60H aug 2, 2025 | 575.30H 21.10M | 5272.60H | -175.30H |
| OY-OCM Tolerance ±10% | 6 | LH engine/LH magneto - perform 500 hrs insp on component: Magneto LH pos (L/H) Make:Slick/Champion Model: Part Number:4372 Serial Number:12020086 Tracking #:3329 | 7400 | Ins | 500.00H | 4775.30H mar 13, 2025 | 672.60H 25.73M | 5275.30H | -172.60H |
| OY-OCM Tolerance ±10% | 8 | LH engine/RH magneto - perform 500 hrs insp on component: Magneto RH pos (R/H) Make:Slick/Champion Model: Part Number:4370 Serial Number:13010143 Tracking #:5417 | 7400 | Ins | 500.00H | 4872.60H aug 2, 2025 | 575.30H 21.10M | 5372.60H | -75.30H |
| OY-OCM | 15 | LH vacuum pump - Vane wear insp on component: Vacuum pump (L/H) Make:RAPCO Model:Wear port equipped Part Number:RAP441CC-7 Serial Number:C5060 Tracking #:16865 | 3700 | Ins | 100.00H 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5047.90H feb 9, 2027 | -400.00H -2.87M |
| OY-OCM | MASTER-15 | Perform Propeller Balancing Acc Propeller Owners Manual 115N | 6100 | Cal | 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | feb 9, 2027 | -2.87M |
| OY-OCM Tolerance ±8% | MASTER-30 | R/H Engine - Engine internal corrosion inspection, TBO extension. | 7100 | Ins | 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | feb 9, 2027 | -2.87M |
| OY-OCM Tolerance ±0% | MASTER-28 | R/H Engine, Additional task (Inspection) on the engine when it is extended.(Inspection) | 7100 | Ins | 100.00H 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5047.90H feb 9, 2027 | -400.00H -2.87M |
| OY-OCM Tolerance ±0% | MASTER-10 | RH Engine - Valve inspection ref. to AMM SPL A | 7100 | Ins | 400.00H | 4872.60H aug 2, 2025 | 575.30H 21.10M | 5272.60H | -175.30H |
| OY-OCM Tolerance ±10% | 10 | RH engine/LH magneto - perform 500 hrs insp on component: Magneto LH pos (R/H) Make:Slick/Champion Model: Part Number:4372 Serial Number:11082201 Tracking #:3118 | 7400 | Ins | 500.00H | 4872.60H aug 2, 2025 | 575.30H 21.10M | 5372.60H | -75.30H |
| OY-OCM Tolerance ±10% | 12 | RH engine/RH magneto - perform 500 hrs insp on component: Magneto RH pos (L/H) Make:Slick/Champion Part Number:4370 Serial Number:13010144 Tracking #:5420 | 7400 | Ins | 500.00H | 4872.60H aug 2, 2025 | 575.30H 21.10M | 5372.60H | -75.30H |
| OY-OCM Tolerance ±10% | 14 | RH vacuum pump - Vane wear insp on component: Vacuum pump (R/H) Part Number:RAP441CC-23 Serial Number:C4752 Tracking #:14524 | 3700 | Ins | 100.00H 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5047.90H feb 9, 2027 | -400.00H -2.87M |
| OY-OCM | MASTER-24 | Seat belt and shoulder harness check of contions | 2500 | Ins | 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | feb 9, 2027 | -2.87M |
| OY-OCM | MASTER-23 | Stand by Compass calibration | 3400 | Cal | 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | feb 9, 2027 | -2.87M |

| AD's |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reg. | AD No. | Description | Type | Perf. | TTSC | Due | Rem. |
| OY-OCM | AD 2015-19-07 Engine | Lycoming Engines fuel injected reciprocating engine models, L/H Engine. Engine (L/H) L-19727-51A | Recur | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5047.90H | -400.00H |
| OY-OCM | AD 2015-19-07 Engine | Lycoming Engines fuel injected reciprocating engine models, R/H Engine. Engine (R/H) L-19728-51A | Recur | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5047.90H | -400.00H |
| OY-OCM | EAD 2021-0267 (PAD 21-158) Tolerance ±0% | RUDDER HINGES. | Recur | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5147.90H feb 9, 2027 | -300.00H -2.87M |
| OY-OCM | FAA AD 2024-21-02 Engine | Connecting Rod Assy Inspection. | Recur | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5002.90H | -445.00H |
| OY-OCM | LYC SB 650 Engine | Intake Pipe and Flange Inspection. | Recur |  |  | 4997.90H feb 3, 2027 | -450.00H -3.07M |
| OY-OCM | RAI 96-032 | Insp of Wing box area (SB 93). | Recur | 4775.30H mar 13, 2025 | 672.60H 25.73M | 5275.30H | -172.60H |
| OY-OCM | RAI AD 1991-125 Engine | Exhaust muffler insp (SB 83). | Recur | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5147.90H | -300.00H |
| OY-OCM | RAI AD 2002-415 AIRFRAME GROUP | Fuel tank selector insp (SB 113). | Recur | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5047.90H | -400.00H |
| OY-OCM | RAI PDA 85-135, IT-1987-36 AIRFRAME GROUP | Wing leading edge (SB 67 R1). | Recur | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5047.90H | -400.00H |
| OY-OCM | RAI PDA 92-155 AIRFRAME GROUP | Elevator torque tube horn attachment (SB 87 R1). | Recur | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5147.90H | -300.00H |
| OY-OCM | RAI PDA 94-026 AIRFRAME GROUP | Flight control shelves (SB 91). | Recur | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5147.90H feb 9, 2027 | -300.00H -2.87M |
| OY-OCM | RAI PDA 96-337 AIRFRAME GROUP | Flight Control cables - 100hrs insp (SB 75 R3). | Recur | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5047.90H | -400.00H |
| OY-OCM | RAI PDA 96-337 AIRFRAME GROUP | Flight control cables - 400 hr insp (SB 75 R3). | Recur | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5347.90H | -100.00H |
| OY-OCM | SB 70 REV 2 AIRFRAME GROUP | ENG MOUNT/WING ATTACHMENT, section 3 inspection. | Recur | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5447.90H feb 9, 2028 | 0.00H 9.13M |
| OY-OCM | SB 74 AIRFRAME GROUP | ELECTRICAL SYSTEM. | Recur | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5447.90H feb 9, 2028 | 0.00H 9.13M |

| Inspections |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reg. | Name | Limit | Perf. | TTSC | Due | Rem. |
| OY-OCM | Insurance Certificate | 12.00M | 4775.30H mar 4, 2025 | 672.60H 26.03M | okt 31, 2025 | -18.13M |
| OY-OCM | AMP review | 12.00M | 4775.30H jan 20, 2025 | 672.60H 27.50M | jan 20, 2026 | -15.50M |
| OY-OCM | ARC Reneval | 12.00M | 4792.10H jun 10, 2025 | 655.80H 22.83M | jun 16, 2026 | -10.63M |
| OY-OCM | 50 hrs Inspection P68B & C Tolerance ±10% | 50.00H 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | 4997.90H feb 9, 2027 | -450.00H -2.87M |
| OY-OCM | 100 hrs Inspection P68B & C Tolerance ±10% | 100.00H 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5047.90H feb 9, 2027 | -400.00H -2.87M |
| OY-OCM | Cylinder pressure test | 100.00H | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5047.90H | -400.00H |
| OY-OCM | Check for new dent and note it on the "bulekort" | 100.00H | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5047.90H | -400.00H |
| OY-OCM | 100 Hrs MT propeller insp Tolerance ±0% | 100.00H 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5047.90H feb 9, 2027 | -400.00H -2.87M |
| OY-OCM | 100 hrs Lubrication P68B | 100.00H 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5047.90H feb 9, 2027 | -400.00H -2.87M |
| OY-OCM | SPL H insp P68B & C Tolerance ±0% | 200.00H 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5147.90H feb 9, 2027 | -300.00H -2.87M |
| OY-OCM | 200 hrs inspection P68B & C Tolerance ±10% | 200.00H 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5147.90H feb 9, 2027 | -300.00H -2.87M |
| OY-OCM | 200 hrs Lubrication P68B Tolerance ±0% | 200.00H 12.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5147.90H feb 9, 2027 | -300.00H -2.87M |
| OY-OCM | 500 hrs Lubrication P68B Tolerance ±0% | 500.00H | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5447.90H | 0.00H |
| OY-OCM | SPL F insp P68B & C Tolerance ±0% | 500.00H | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5447.90H | 0.00H |
| OY-OCM | SPL B insp P68B & C Tolerance ±0% | 500.00H 24.00M | 4947.90H feb 9, 2026 | 500.00H 14.87M | 5447.90H feb 9, 2028 | 0.00H 9.13M |
| OY-OCM | SPL C insp P68B & C Tolerance ±0% | 1000.00H 48.00M | 4416.10H apr 25, 2023 | 1031.80H 48.33M | 5416.10H apr 25, 2027 | -31.80H -0.33M |
| OY-OCM | SPL G insp (Eng mount bolts insp.) P68B & C Tolerance ±0% | 2000.00H 48.00M | 4416.10H apr 25, 2023 | 1031.80H 48.33M | 6416.10H apr 25, 2027 | 968.20H -0.33M |

| Defects |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | REG | # | ATA | Description | Correction | Date | Due Date | TTAF |
| Deferred | OY-OCM | 1 | 3000 | Airframe de-ice system u/s, Timer defect. Previous correction: | DEFFERED. | jun 28, 2022 | dec 31, 2025 | 4650.00 |

Forecast Cost

Component list. REG; Component/Action; Occ.(Max); Part; Labour; Misc; Total. OY-OCM; Air intake filter (L/H) / Rep; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Air intake filter (R/H) / Rep; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Antenna ADF / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Antenna COM1 / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Antenna COM2 / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Antenna DME / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Antenna Marker Beacon / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Antenna Transponder / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Antenna UHF / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Carbon Monxide Gas detector (CO) (Cockpit) / Rep; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; De-ice sytem / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Engine (L/H) / Ove; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Engine (R/H) / Ove; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Engine tubular mount (L/H) / Ove; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Engine tubular mount (R/H) / Ove; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; FIRST AID KIT NIVEAU 3 (AOC) / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Fuel Injector (L/H) / Ove; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Fuel Injector (R/H) / Ove; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Fuel Pump (L/H) (L/H) / Ove; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Fuel Pump (R/H) (R/H) / Ove; 1.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Instrument airfilter / Rep; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Instrument airfilter (For A/P attitude ind) / Rep; 1.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Magneto LH pos (R/H) / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Nav, Strob and landing Light system / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; PROP DE-ICE TIMER / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; PROP DEICE BRUSH BLOCK (L/H) / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; PROP DEICE BRUSH BLOCK (R/H) / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Rudder /; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Rudder trim actuator (Tail) / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Stabilator trim actuator / Rep; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Start Booster (L/H) / Ins; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Start Booster (R/H) / Ins; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Strobe light box / On; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Total; 0.00$; 0.00$; 0.00$; 0.00$. Tasks List. REG; Task; Occ.(Max); Part; Labour; Misc; Total. OY-OCM; Annual check of Garmin GNS 530W ref. to Gama Aviation ICA doc. DQ-14-263 last rev. (Issue 2); 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Annual inspection of ELT; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Antenna inspection.; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Check of emergency equipment FIRST AID KIT, Extinguishers, Life Jacket; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; De-ice boots: Add age 1 master; 3.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; De-ice boots: Add Icex II; 10.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; L/H Engine - Engine internal corrosion inspection, TBO extension.; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; L/H Engine, Additional task (Inspection) on the engine when it is extended.(Inspection); 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; LH Engine - Valve inspection ref. to AMM SPL A; 1.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; LH engine/LH magneto - perform 500 hrs insp; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; LH engine/RH magneto - perform 500 hrs insp; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; LH vacuum pump - Vane wear insp; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Perform Propeller Balancing Acc Propeller Owners Manual 115N; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; R/H Engine - Engine internal corrosion inspection, TBO extension.; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; R/H Engine, Additional task (Inspection) on the engine when it is extended.(Inspection); 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; RH Engine - Valve inspection ref. to AMM SPL A; 1.4; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; RH engine/LH magneto - perform 500 hrs insp; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; RH engine/RH magneto - perform 500 hrs insp; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; RH vacuum pump - Vane wear insp; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Seat belt and shoulder harness check of contions; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Stand by Compass calibration; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Total; 0.00$; 0.00$; 0.00$; 0.00$. AD's List. REG; AD's; Occ.(Max); Part; Labour; Misc; Total. OY-OCM; AD 2015-19-07; 5.0; 0.00$; 1 845.00$; 0.00$; 1 845.00$. OY-OCM; AD 2015-19-07; 5.0; 0.00$; 1 845.00$; 0.00$; 1 845.00$. OY-OCM; EAD 2021-0267 (PAD 21-158); 2.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; FAA AD 2024-21-02; 9.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; LYC SB 650; 99.1; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; RAI 96-032; 1.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; RAI AD 1991-125; 2.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; RAI AD 2002-415; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; RAI PDA 85-135, IT-1987-36; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; RAI PDA 92-155; 2.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; RAI PDA 94-026; 2.5; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; RAI PDA 96-337; 5.0; 0.00$; 7 380.00$; 0.00$; 7 380.00$. OY-OCM; RAI PDA 96-337; 1.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; SB 70 REV 2; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; SB 74; 1.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Total; 0.00$; 11 070.00$; 0.00$; 11 070.00$. Inspection List. REG; Inspection; Occ.(Max); Part; Labour; Misc; Total. OY-OCM; Insurance Certificate; 2.2; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; AMP review; 2.3; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; ARC Reneval; 1.9; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; 50 hrs Inspection P68B & C; 10.0; 21 600.00$; 73 800.00$; 0.00$; 95 400.00$. OY-OCM; 100 hrs Inspection P68B & C; 5.0; 0.00$; 40 590.00$; 0.00$; 40 590.00$. OY-OCM; Cylinder pressure test; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; Check for new dent and note it on the "bulekort"; 5.0; 0.00$; 3 690.00$; 0.00$; 3 690.00$. OY-OCM; 100 Hrs MT propeller insp; 5.0; 0.00$; 0.00$; 0.00$; 0.00$. OY-OCM; 100 hrs Lubrication P68B; 5.0; 0.00$; 7 380.00$; 0.00$; 7 380.00$. OY-OCM; SPL H insp P68B & C; 2.5; 0.00$; 2 767.50$; 0.00$; 2 767.50$. OY-OCM; 200 hrs inspection P68B & C; 2.5; 0.00$; 14 760.00$; 0.00$; 14 760.00$. OY-OCM; 200 hrs Lubrication P68B; 2.5; 0.00$; 461.25$; 0.00$; 461.25$. OY-OCM; 500 hrs Lubrication P68B; 1.0; 0.00$; 1 845.00$; 0.00$; 1 845.00$. OY-OCM; SPL F insp P68B & C; 1.0; 0.00$; 2 214.00$; 0.00$; 2 214.00$. OY-OCM; SPL B insp P68B & C; 1.0; 0.00$; 2 583.00$; 0.00$; 2 583.00$. OY-OCM; SPL C insp P68B & C; 1.0; 0.00$; 1 476.00$; 0.00$; 1 476.00$. OY-OCM; SPL G insp (Eng mount bolts insp.) P68B & C; 1.0; 0.00$; 10 332.00$; 0.00$; 10 332.00$. OY-OCM; Total; 21 600.00$; 161 898.75$; 0.00$; 183 498.75$. Total. REG; Part; Labour; Misc; Total. OY-OCM; Total; 21 600.00$; 172 968.75$; 0.00$; 194 568.75$.

| This report has been prepared by: |  |  |
| --- | --- | --- |
| Signature | Authorization Number | Date |
| © Aircraft Maintenance Systems RD inc |  |  |